Title: Hardware Acceleration of Imaging Algorithm for Stroke Monitoring.

Name: Imran Sarwar
Current Affiliation: Balochistan University of Engineering & Technology Khuzdar, Balochistan, Pakistan.


Abstract: (Within 250 words)

Brain stroke is a critical problem affecting people worldwide and is among the common cause of permanent disability or death worldwide with more than 15 million people getting affected annually.

During brain stroke, abnormal blood flow inside the head of the patient starts, which can result in devastating changes in the brain or, at worst, lead to death. On-the-spot accurate detection and monitoring employing head imaging in the post-acute stage are of importance for appropriate medication for ensuring the proper care and complete recovery of the patient. Currently available standard imaging tools such as X-Ray Mammography, MRI and CT, the most common stroke monitoring techniques, are limited due to their cost, size, use of ionizing radiations and low reliability. These limitations encouraged us to design alternative imaging techniques, to overcome their limitations and to get reliable results.

The work presented here is in the framework of designing highly efficient, low cost, low power, and reliable Microwave imaging (MI) system to monitor brain stroke. MI systems have the advantage of using low ionizing radiations, low cost, ease of use and can produce reliable results.

The designed MI subsystem for stroke monitoring, is a low cost, low power and portable system. The imaging algorithm used is Truncated Singular Value Decomposition (TSVD) in which signals are acquired at a single frequency, and the execution of the algorithm for image reconstruction is much faster. The algorithm detects the variation of the blood mass across two successive data, and then the reconstructed image shows this variation.

Moreover, the imaging algorithm is accelerated due to the implementation of hardware accelerator in the FPGA of the Zynq SoC and the obtained results are at least five times faster than the software execution of the algorithm.

Biography: (Up to 100 words)

Imran Sarwar age 29 is Currently a PhD researcher at Politecico di Torino, Italy. In March 2019 he is hopeful to get his PhD degree. He is from BUET Khuzdar, Balochistan, Pakistan. After his BE in BUETK he was awarded scholarship for MS and PhD in Politecnico, Italy. His research work included microwave imaging, hardware acceleration of imaging algorithms for biomedical applications.

Presenting author details:

Full Name which you wish to be printed: 
Email ID: eng.imransarwar@yahoo.com
Contact Tel Number: +393293850827, +923333345994
Twitter/ Linked account:
Session of Interest: Oral presentation/ Poster presentation
Category: (Oral presentation/ Poster presentation)