Ultra-trace analysis of D-and L-aspartic acid applying one-by-one approach on a dual imprinted electrochemical sensor

Bhim Bali Prasad

Analytical Division,

Department of Chemistry,

Institute of Science,

Banaras Hindu University,

Varanasi 221005, India

Abstract

An enantiomeric pair imprinted polymeric film, with embedded functionalized multiwalled carbon nanotubes, was developed following the ‘surface grafting from’ approach onto a gold nanoparticles decorated pencil graphite electrode. Double imprinting of chiral molecules in a single polymer motif, as a sensory platform, for the one-by-one evaluation of individual components of a racemic mixture, is a challenging task. The underlying state-of-art proposed for this purpose is novel and if this works well, any two simple molecules could be used as templates. In this work, a pencil graphite electrode was first dipped for the overnight in the aqueous suspension of gold nanoparticles. The electrode was then subjected to spin coating with a pre-polymerization mixture consisting a monomer (N-acryloyl-pyrrolidine-2,5-dione) , templates (D- and L-Aspartic acid) , a cross-linker (ethylene glycol dimethacrylate) in the presence of an initiator (α,α’-azoisobutyronitrile). As aspartic acid isomers have been found to be electro-inactive, their evaluation was feasible indirectly with the help of a potassium ferricyanide probe. The quantitation ability of the proposed sensor, with differential pulse anodic stripping voltammetric transduction, was found to be in the tune of 4.08 ng mL−1 (S/N=3) for both the enantiomers in the real samples, which reportedly manifest several chronic diseases at their stringent limits.

E-mail id: prof.bbpd@yahoo.com

Phone +91 9451954449

Fax: +91 542 22368127