Determination of benzene, toluene, ethylbenzene and xylene in fi eld and laboratory by means of cold fi ber SPME equipped with thermoelectric cooler and GC/FID method

Leila Tajik1, Abdulrahman Bahrami1, Alireza Ghiasvand2, Farshid Ghorbani Shahna1

1Hamadan University of Medical Sciences, Center of Excellence for Occupational Health, School of Public Health,

Hamadan, Iran

2Lorestan University, Department of Chemistry, Faculty of Science, Khoramabad, Iran

*Corresponding author: e-mail: bahrami@umsha.ac.ir

A simple and effective cooling device based on a thermoelectric cooler was applied to cool the SPME fi ber. The

device was used for quantitative extraction of aromatic hydrocarbons in the air. Several factors such as coating

temperature, extraction temperature and relative humidity in the laboratory setting were optimized. Comparison

of the results between the cold fi ber SPME (CF-SPME) and NIOSH 1501 method on standard test atmosphere

indicated a satisfactory agreement. The CF-SPME and SPME method were also compared. The results revealed

that CF-SPME has the most appropriate outcome for the extraction of aromatic hydrocarbons from the ambient

air. The cold fi ber SPME technique showed good results for several validation parameters. Under the optimized conditions, the limits of detection (LOD) and the limits of quantifi cation (LOQ) ranged from 0.00019 to 0.00033 and 0.0006 to 0.001 ng ml–1, respectively. The intra-day relative standard deviation (RSD) showed ranging from 4.8 to 10.5%.

Keywords: aromatic hydrocarbons, air sampling, cold fi ber, solid-phase microextraction.