Title: Predicting the Deformation of a Sloping Tunnel: Biharmonic Evaluation

Name

University of Science and Technology Beijing, China

Abstract (300 Words)

When excavating a tunnel, the stresses are distributed asymmetrically along the tunnel cross-section. Other factors, particularly slope friction force and excavation speed, can also contribute to the deformation and displacement of a tunnel. Despite this, several authors have used the complex potential method to predict the ground deformation surrounding the tunnel. However, their applicability to the ground response caused by the asymmetric stress distribution around the tunnel wall is analysed in this context. This paper, therefore, proposes an approximate solution on the slope to predict the tunnel cross-section deformation. The solution is based on the biharmonic method to predict analytically and numerically the ground deformation around the tunnel. However, the deformation parameters established take into account the friction force and the ratio between the radius R and the depth h of the tunnel, which induces an asymmetric redistribution of the stresses at the boundary conditions. Data from the Qijiazhuang tunnel case are used to justify the proposed analytical solutions. A hypothesis of rupture of the central line of the tunnel is considered, taking into account the andesitic nature of the soil. This solution is an essential guide for analysing deformations in complex geological conditions and structures, such as steeper slopes.

Recent Publications

- Jong, Y. -G., Liu, Y., Chen, Z., Mabe Fogang , P.  (2021). “Hypoplastic Interface Model considering Plane Strain Condition and Surface Roughness.” Advances in Civil Engineering, 13.  https://doi.org/10.1155/2021/1473181.

- Mabe Fogang, P.; Liu, Y.; Zhao J, l. (2022); Azeuda Ndonfack, K.I. Soil Deformation around a Cylindrical Cavity under Drained Conditions: Theoretical Analysis. Adv. Civ. Eng. 2022, 4127660. https://doi.org/10.1155/2022/4127660.

- Mabe Fogang, P.;  Liu, Y.; Zhao J. –L.;  Ka, T. A. and Xu, S. (2023). Analytical Prediction of Tunnel Deformation Beneath an Inclined Plane: Complex Potential Analysis. Appl. Sci. 13(5), 3252; https://doi.org/10.3390/app13053252

Biography (100 Words)

My name is Pieride Mabe Fogang, and I'm from Cameroon. I'm the third born in a polygamous family of 18 children. I started school at three at Djeleng 5 primary school in Bafoussam. I enrolled at the University of Yaoundé I, Department of Earth Sciences, where I obtained a Bachelor's degree in 2013 and a Master's degree in 2016 with honours. I interned at the Memev'ele dam construction project as a geotechnical engineer for three years. I then obtained a scholarship for a PhD in 2019 at the Beijing University of Science and Technology in China.

Email : mabe.pieride@yahoo.com
Contact Info (WhatsApp Number): +86 18810158173

Photograph