# Maximizing the Efficiency of Bioenergy Generation: Designing Mechanism of Biofuel and Electricity Production with Rhodopseudomonas palustris

Grace Li, Great Lake Bioenergy Research Center, USA

### Abstract

Rhodopseudomonas palustris is a photosynthetic bacterium that encompasses four different modes of metabolism. One mode is extremely valuable for its ability to digest lignocellulose and produce furan-containing fatty acid(FFA) which has applications in pharmaceuticals, biofuels and industrial solvents. Additionally, furan derivatives and phenolic compounds (byproducts of FFA) are proven to be electron donors for electricity generation through R,palustris.  Microbial Fuel Cell (MFC)is a promising approach for renewable energy research, and non-straight-chain fatty acids are valuable in many ways if generated in large quantities. Here in this paper, we propose a novel mechanism to effectively recombine these two features of R.palustris. Glucose-rich medium that also contains furan derivatives is used to grow cell culture at the anode. Carbon cathode and platinum wires will be used to transfer electrons from anode to cathode where they will subsequently be consumed by R.palustris mutants that synthesize di-methyl-furan-fatty acid. Methods including genetic engineering are used to create ∆ChrR mutants that will amplify the enzymes in this pathway. Production of the products will theoretically be facilitated by the change in electrical potential across the cell membrane as the bacteria accept electrons from the anode. Furan derivatives will be fed back into the anode chamber externally in the reactor.

### Image

Figure 1.                                 Figure.2

Figure1. Electrons generated from the oxidation of furan derivatives at the anode will flow toward the cathode where R.palustris will use them to catalyze dimethyl-FFA productions. Both chambers operate in anaerobic conditions under light.

Figure2.In this pathway of synthesizing dimethyl-furan –fatty-acid in R.palustris, the potential genes responsible for FFA synthesis are clustered together in an operon. Electrons received from the anode will function as a base catalyst (e.g. sodium methoxide in transesterification) to make the carbonyl group more nucleophile and facilitate the initiation of this pathway.

Recent Publications (minimum 5)

1. Mueller S, et al. (2008) General detoxification and stress responses are mediated

by oxidized lipids through TGA transcription factors in Arabidopsis. Plant Cell 20(3):

768–785.

2. Cronan JE (2002) Phospholipid modifications in bacteria. Curr Opin Microbiol 5:202–205.

3. Cronan JE (2003) Bacterial membrane lipids: Where do we stand? Annu Rev Microbiol

57(1):203–224.

4.Bajracharya, S.ab,  Sharma, M.ac (2016). “An overview on emerging bioelectrochemical systems (BESs): Technology for sustainable electricity, waste remediation, resource recovery, chemical production and beyond.” Renewable Energy. 98.153-170

5. Lemke RAS(2014). Synthesis and scavenging role of furan fatty acids. PNAS 1405520111

6. Wayne S. Kontur, Eva C. Ziegelhoffer, Melanie A. Spero, Saheed Imam, Daniel R. Noguera, Timothy J. Donohue (2011). " Pathways Involved in Reductant Distribution during Photobiological H2 Production by Rhodobacter sphaeroides." Appl. Environ. Microbiol. 20.7425-7429

Biography (150 word limit)

Email: jli683@wisc.edu

Grace Li has been contributing to the forefront of Bioenergy research through her innovative work in Biofuel, Bioelectricity, and Energy System Integration. Grace works as a research fellow in the Great Lakes Bioenergy Research Center which recently received $250 million funding from the US Department of Energy for the next five fiscal years. Grace is also in the process of patenting her designs and eventually scale them up to industrial level productions.

Notes/Comments: