Biodiesel and diesel engine: correlation between biodiesel specific mass and the engine consumption and the pollutants emission

Osmano Souza Valente D.Sc. Pontifícia Universidade Católica de Minas Gerais-  Brazil

Sérgio de Morais Hanriot, DSc. Pontifícia Universidade Católica de Minas Gerais - Brazil

Deliene Costa, MSc, Pontifícia Universidade Católica de Minas Gerais - Brazil

### Abstract

The fuel used in diesel cycle engines is diesel oil, a mixture of mineral hydrocarbons. Biodiesel is a compound of fatty acids obtained from vegetable oils and animal fat in the transesterification process. In Brazil, the National Petroleum Agency (ANP) regulates the characteristics of these fuels. The characteristics of both diesel oil and biodiesel depend on the composition of them. This work presents the composition of biodiesel from two oil seeds and the average composition of diesel oil, highlights the influence of the composition of these oils on the specific mass and viscosity and presents the consumption and emission of a diesel engine, constituting a motor group and electricity generator, fed with these fuels. Soybean biodiesel had a specific mass of 3.6% higher when compared to diesel oil, and biodiesel of castor bean 6.77%. The consumption of the engine with zero load and mixed feed containing 35% was higher 5.39% with soybean biodiesel and 9.97% with castor biodiesel. With these percentages of biodiesel in the fuel, there was an increase in CO, NOx, unburned hydrocarbons and soot emissions. There were no significant variations in CO2 emissions.

1. De Morais, André Marcelino; Mendes Justino, Marco Aurélio ; Valente, Osmano Souza ; Hanriot, Sérgio De Morais ; Sodré, José Ricardo . Hydrogen impacts on performance and CO2 emissions from a diesel power generator. International Journal of Hydrogen Energy, v. 38, p. 6857-6864, 2013..

2. Reis, Mara N.E. ; Hanriot, Sergio . Incompressible Pulsating Flow For Low Reynolds Numbers In Orifice Plates. Flow Measurement and Instrumentation, v. 54, p. 146-157, 2017.

3. Hanriot, Sergio De Morais; Guimarães, Deliene Costa ; Valente, Osmano Souza ; Maia, Cristiana Brasil . THE investigation of ignition delay times of t-fuel and u-fuel mixtures using shock tube with flame detection. In: 12th International Conference on Heat Transfer, Fluid Mechanics and Thermodynamics, 2016, Malaga. 12th International Conference on Heat Transfer, Fluid Mechanics and Thermodynamics, 2016.

4. De Vasconcellos, Guilherme L.F. ; De Queiroz, Matheus D. ; Ledo, Luiz F.R. ; Hanriot, Sérgio De M. ; Maia, Cristiana Brasil . Determination of the Diameter of a Cylinder to Insertion in a Low-Speed Wind Tunnel Based on EFD and CFD. Applied Mechanics and Materials, v. 798, p. 165-169, 2015

5. GUZELLA, Matheus dos Santos ; Cabezas-Gómez, Luben ; DA SILVA, JOSÉ ANTÔNIO ; MAIA, Cristiana Brasil ; HANRIOT, SÉRGIO DE MORAIS . Numerical modeling of the thermal-hydraulic behavior of wire-on-tube condensers operating with HFC-134a using homogeneous equilibrium model: evaluation of some void fraction correlations. Heat and Mass Transfer, v. 52, p. 183-195, 2016.

Biography (150 word limit)

He holds a PhD in Mechanical Engineering in the area of ​​Thermal and Fluids by the Federal University of Minas Gerais (2001). He has done part of the studies at FIAT Research Center (CRF) in Turin, Italy. The master's degree in Mechanical Engineering was held at the Federal University of Minas Gerais (1990) in the area of Thermal and Fluids. Graduation in Physics (1987) was held at the Federal University of Minas Gerais. He is adjunct professor IV of the Post-Graduate Program in Mechanical Engineering of the Pontifical Catholic University of Minas Gerais. He has experience in the field of Mechanical Engineering, with emphasis on Thermal and Fluids, working mainly on the following topics: internal combustion engines, pulsed flow, applied fluid dynamics, refrigeration, compressible flow and waves.

Email: hanriot@pucminas.br

Notes/Comments: