Photograph

Title of the Abstract: Biodegradation of Trichloroethylene by acclimated microbiota in a 			batch reactor

Author Name: 	Divya Baskaran

Affiliation: 		Ph. D Research Scholar

University:		Annamalai University

Country: 		India

Abstract:

Air pollution has become a topic of increasing environmental concern in many cities all over the world. Trichloroethylene (TCE) is one of the most recalcitrant pollutants in environmental system because it frequently reported to air, soil and groundwater contaminant. In earlier, a different microorganism has been investigated for biodegradation of trichloroethylene. In this present work, an attempt has been made to compost of turkey waste capable to degrade the TCE by aerobically. Batch biodegradation was conducted by mixed cultures over a TCE concentration ranging from 25 to 250 ppm at neutral pH. The specific growth rate was increased with increase in substrate concentration and then decreased with increase in TCE concentration when substrate inhibition obtained. At high concentration of TCE affects the degradation patterns due to the formation of acid metabolites such as chloral, glyoxylate and formate which all are unstable and diminished at shortly. However, an increasing chloride ion during degradation it directly reflects the TCE degraded by the mixed culture. By fitting the different substrate inhibition models, biokinetic constants are necessary to understand the kinetics of biodegradation process were evaluated. Biochemical test and physical characteristic test were carried out for mixed microbiota.

Biography:

Ms. Divya Baskaran is a research scholar of department of chemical engineering in Annamalai University. Now, she was working as a JRF in the UGC funded project on “Biofiltration for the removal of Volatile Organic Compounds” since July 2016. She had four publications and one conference proceedings in their research area. Recently, she got best session paper award in presentation at Singapore conference.  Work in the field of waste (water and gas) treatment by biological reactors.

Research Interests:

Environmental Biotechnology

References:

Dr. R. Ravi

Associate Professor of Chemical Engineering

Annamalai University

Mail Id:lect_ravi@yahoo.com

Contact Number: +91 9444290857

Suggestions of colleagues/Professors/scholars to send invitation (if any)

Suggestions of Sessions to the conference (if any)