Analysis of NTCP Based Radiobiological Models:  A Systematic Review of Literature

Rahman M1, Kausar A1, Azhari HA1, Zakaria GA1, 2.

1Dept of Medical Physics and Biomedical Engineering, Gono Bishwabidyalay, Savar, Dhaka , Bangladesh.

1,2Dept. of medical Radiation Physics , Gummersbach Hospital , Academic Teaching Hospital of the University of Cologne, Gummersbach , Germany.

ABSTRACT

Background and purpose: The aim of radiotherapy is to deliver maximum dose of radiation to destroy the tumor cells while at the same time minimizing the damage to normal healthy tissues. To achieve the optimal treatment goal, radiobiological parameters are to evaluate and predict the outcome of this treatment plan in terms of both a tumor control probability (TCP) and a normal tissue complication probability (NTCP). Different types of radiobiological model are using to achieve prescribed treatment dose of radiation during the tumor control. Where Tumor Control Probability (TCP) models plays a important role in order to achieve desired dose to the tumor.  A suitable NTCP model is theoretically found among different models that can be used in treatment plan evaluation.

Materials and Methods: Six NTCP mathematical Radiobiological models were analyzed in this project theoretically. Lyman–Kutcher–Burman ,Critical element ,critical volume, Relative Seriality, parallel architecture, Weibull distribution Model were analyzed from the derivation .So that each model was discussed elaborately with its various parameters which was used in the calculation of normal tissue complication probability  during the treatment in Radiotherapy as well as compared among them.

Results: Since all complicated NTCP model predict same as the simple NTCP model that is Lyman–Kutcher–Burman model as well as it is computationally efficient. Also Lyman–Kutcher–Burman model can be used in different treatment planning system incorporating with other model. For this reason our suggested model is Lyman–Kutcher–Burman NTCP model which can be used in treatment plan evaluation/verification.

Conclusion: After analyzing number of NTCP model we came to conclude that Lyman–Kutcher–Burman model is the suitable model for treatment plan evaluation. On the hand other model can be used when absolutely necessary.

Keywords: Radiobiological Model, Tumor control probability (TCP), Normal tissue complication probability (NTCP), NTCP Model.