STUDY MODELING OF ISOTHERM KINETICS AND CAPACITY ADSORPTION OF METHYLEN BLUE DYE ONTO ZEOLITE L SYNTHESIZED FROM SILICA EXTRACTION BY RICE HUSK ASH OF HYFROTHERMAL METHOD

Atthar Luqman Ivansyah1; Soni Setiadji2; Denia Febby Nurbaeti3: Ira Novianti4; Ai  Nurohmah5

The researcher are using Tetraethylortosilica (TEOS) as a silica source for the synthesis of zeolite. Rice husk is by-product of the rice milling industry. Utilization of rice husk as silica source TEOS bias as an alternative replacement. Inorganic compounds contained in the rice husk ash silica (SiO2) with a composition of 94%. Zeolite L synthesized using silica rice husk ash as silica source with hydrothermal method at 170ᵒC temperatures for 24 hours. Zeolite L crystal were identified using X-Ray Diffraction (XRD). And then test the adsorption capacity using methylene blue dye with parameters: time contact, pH, and initial dye concentration. Adsorption capacity increased with increasing initial dye concentration and pH. Further FTIR (Fourier Transform Infra-Red) analysis and SEM (Scanning Electron Microscopy) to see the bonding compound and morphology of zeolite L before and after adsorption. Adsorption process followed the Langmuir isotherm models with the highest efficient (R2 = 0,9969) at 298 K. while the kinetic model of adsorption followed pseudo-second-order rate with the highest coefficient (R2 = 0,9943).

Keywords: zeolite L; methylene blue dye; adsorption isotherm; adsorption kinetic