Title: Hybrid flexible plasmonic SERS substrate with improved assemblage of Ag@SiO2 nanocubes on a miniaturized paper platform for detection of Melamine

Menbere Leul Mekonnen a, Wei-Nien Sub, Ching-Hsiang Chen b, Bing-Joe Hwanga,c,*

a Department of Chemical Engineering, National Taiwan University of Science and Technology, Taipei 106, Taiwan

b Graduate Institute of Applied Science and Technology, National Taiwan University of Science and Technology, Taipei 106, Taiwan

c National Synchrotron Radiation Research Centere, Hsinchu, Taiwan

Abstract

Surface Enhanced Raman Scattering (SERS), benefitted by its fingerprinting ability of molecules can be a reliable option for trace analysis in the food matrix. Since the sensitivity of a SERS method is highly dependent on the degree, to which the Raman signature is enhanced, fabrication substrate that could adequately amplify the local field through excitation of localized surface plasmon resonances (LSPRs) is critical. Hence, control over the nanostructures morphology and improving their inter-particle distance is important for obtaining improved SERS activity. In addition, flexible platforms like paper offer the means for improving the nanostructures assemblage with ease of fabrication at low cost and improved sample collection efficiency.

Here in, inexpensive, flexible paper-plasmonic hybrid SERS substrate is presented by loading Ag@SiO2 nanocubes on a miniaturized filter paper through vacuum filtration. The miniaturized sensing platform owes a reasonable distribution and inter-particle spacing of nanocubes. The fibrous structure of paper promotes the assemblage of sharp-edged nanostructures which significantly improve their distribution and SERS activity.The thin silica shell improved the stability and inter-particle spacing of silver nanocubes in the sensing platform, rendering enhanced SERS activity through plasmon-coupling effect as compared to a conventional rigid substrate. Assessment of analytical performances of the substrate for melamine quantification showed a good linearity (R2 = 0.9947) up to 1 mg/L with a limit of detection 0.06 mg/L. The detection limit in liquid milk was down to 0.17 mg/L, which is below the permissible

Keywords— Ag@SiO2 nanocubes, flexible substrate, filter paper, melamine, SERS

Professional Biography (100-150 words)

Menbere Leul Mekonnen has a master degree  in analytical chemistry and now he is a PhD student in the Chemical engineering Department of the National Taiwan University of Science and Technology, under the supervision of Prof. BJ Hwang. He is focusing on SERS and analytical application in food matrix and preparation of plasmonic substrates.

Email Id: menberelm@gmail.com