Using Sandwich Method to Increase the Detection Sensitivity of procalcitonin (PCT) in Fiber Optic Particle Plasmon Resonance Biosensor

Chang-Yue Chiang1,2 , Ting-Chou Chang1,2 , Chia-Wei Hsu2, Chun-Jen Huang3, Tze-Ta Huang4, Lai-Kwan Chau1,2

1 Department of Chemistry and Biochemistry, National Chung Cheng University,Chiayi, 62102, Taiwan.

2Center for Nano Bio-Detection, AIM-HI, National Chung Cheng University, Chiayi, 62102, Taiwan

3Graduate Institute of Biomedical Engineering and Chemical & Materials Engineering Department, National Central University , Jhong-Li, Taoyuan 320, Taiwan

4Institute of Oral Medicine, College of Medicine and Hospital, National Cheng Kung University, Tainan 70101, Taiwan.

Abstract

Septicemia is a serious disease that requires early diagnosis, and procalcitonin (PCT) serves as a diagnostic biomarker for this disease. Therefore, a rapid, sensitive, and cost-efficient platform is urgently needed for septicemia detection. The fiber optic particle plasmon resonance (FOPPR) biosensor based on the integration of gold nanoparticles and fiber optic evanescent wave may provide an ultrasensitive and rapid detection approach. Herein, an ultrasensitive sandwich strategy in the FOPPR sensing platform for PCT detection has been developed. The results demonstrate that the proposed sandwich strategy for PCT provides a wide linear response range from 1 pg/mL to 100 ng/mL and an extremely low limit of detection (LOD) of 0.28 pg/mL (0.021 pM). In addition, the sandwich strategy has advantages of convenience, low-cost, short analysis time (15 min), good specificity, acceptable stability and reproducibility. Finally, the results from the FOPPR biosensor using the sandwich strategy were compared to a clinically validated electrochemiluninescence assay (Roche cobas e411). An excellent correlation coefficient (r = 0.99) between the two methods for blood plasma samples from twelve septicemia patients was achieved, indicating that the FOPPR biosensor using the new sandwich strategy has the potential to be used in actual clinical diagnosis of PCT. Furthermore, it is also potentially applicable to the detection of many other biomarkers, and thus provides an ideal technical tool for point-of-care diagnostics.

Keywords—Septicemia, procalcitonin (PCT), fiber optic particle plasmon resonance (FOPPR) biosensor, gold nanoparticles

Professional Biography

Dr. Chang-Yue Chiang received his PhD in Department of Chemistry and Biochemistry from National Chung Cheng University, Taiwan. His research work has been mainly focused on the design of new optical nanosensors, preparation of nano-functional materials, and applications of nanobiosensors to clinical and agricultural diagnostics.

Email Id: standford_chiang@hotmail.com

Any Comments: I do not have any opinion.