Abstract Template

Abstract Title: Mineral carbonation of modified and non-modified fly ash in an elevated pressure and temperature

K. Zarębska1, P. Zabierowski1, N. Czuma1, P.Baran1, A. Ćwik1,2, K. Świerczek1

1 Faculty of Energy and Fuels, Al. Mickiewicza 30, 30-059 Cracow (AGH University of Science and Technology, Poland)

2 Institute of Energy Technologies Campus Diagonal-Besos, 08019 Barcelona (UPC Universitat Polytecnica de Catalunya, Spain)

Abstract

In the proposed study, mineral carbonation of the fly ash from one of the polish power plants burning brown coal was performed at laboratory scale. Conditions for this process were: pressure range 4-16 bar and temperature 323 K. For the experiment one fly ash was used. In one experimental set raw form of fly ash was used. In the second experimental set in analogical conditions, the same fly ash was used, but specially, “artificially” prepared for the experiment. Preparation of fly ash  included heating of fly ash in air in elevated temperatures to decompose formed carbonates (result of atmospheric carbon dioxide reaction with eg.: non-reacted calcium oxide from desulphurisation process) and experimentally check and compare the results for such prepared material in correlation to raw fly ash from the same source.

Laboratory investigations proved that the carbon dioxide uptake is increasing with pressure in both analysed cases. “Artificial” preparation of fly ash for the experiments did not result in increase of CO2 uptake. The results indicate that no special preparation of fly ash is needed in order to use it as a material for CO2 utilisation.

Keywords—Carbonation, CO2 utilisation, fly ash, fly ash modification

Professional Biography

Katarzyna Zarębska is an associate professor at the Department of Coal Chemistry and Environmental Sciences, Faculty of Energy and Fuels, AGH University of Science and Technology (Poland). In 2003 she received PhD in chemical technology (with honors) at AGH-UST and in 2013 – DSc in the field of engineering at Central Mining Institute in Katowice (Poland). She has wide experience in the area of environmental monitoring and protection, atmospheric chemistry, mineralogy, crystallography. Recently prof. K. Zarebska works on the relations between the properties of bituminous coals and their sorption capacity during sorption/desorption of CO2, studies unconventional gas deposits, in the context of coal gasification and use in production of hydrogen as an universal energy carrier, analyses particular groups of adsorbates in order to find the parameters of porous structure of coal adsorbents having different chemistry.

Email Id: zarebska@agh.edu.pl

Any Comments: