Electrochemical–catalytic reduction of nitrate over Pd–Cu/γAl2O3 catalyst in cathode chamber: enhanced removal efficiency and N2 selectivity

Zhiqiang Zhang, Wenxin Shi*, Fuyi Cui

State Key Laboratory of Urban Water Resource and Environment, School of Municipal and Environmental Engineering, Harbin Institute of Technology, Harbin 150090, People’s Republic of China

Abstract

The Pd–Cu/γAl2O3 catalysts were prepared by impregnation method and introduced into the cathode chamber of a divided electrochemical denitrification cell with two graphite plates as the cathode and anode, to enhance the nitrate removal efficiency and N2 selectivity. The Pd–Cu/γAl2O3 catalysts were characterized by transmission electron microscope (TEM), X–ray diffraction (XRD), specific surface area measurements (BET) and inductively coupled plasma–atomic emission spectrometry (ICP–AES). In the rationally designed electrochemical–catalytic (ECC) system, the as–prepared catalyst could significantly enhance the nitrate degradation rate to 1.08 mg/L at current density of 10 mA/cm2, which was approx. 2.5 times compared with the electrochemical (EC) system without catalysts adding in. Additionally, a higher nitrogen selectivity of 80.37% was obtained under the same experiment condition. The improved performances were likely due to the presence of a catalytic reduction reaction of nitrate with the appropriate amount of hydrogen generated by electrolysis as reductant. Significantly, the current efficiency was calculated and enhanced value of 20% to 40% (depended on current density) was obtained in the ECC process with a catalyst content of 1.0 g/L.

Keywords— Pd–Cu/γAl2O3 catalyst, nitrate, electrochemical–catalytic, current efficiency

Professional Biography

Zhiqiang Zhang is an PhD student from School of Municipal and Environmental Engineering, Harbin Institute of Technology. His research direction is novel catalyst development and application in the field of environmental science and technology, especially in the removal of oxyanion, such as nitrate, perchlorate and bromate from water by catalytic hydrogenation reduction method using Pd- or Pt-based catalyst.

Email Id: zhangzq2013@yeah.net

Any Comments: