Enhancement of aerobic granulation and nutrients removal by an algal-bacterial consortium in a lab-scale photo-bioreactor

Bing Zhang, Wenxin Shi*, Fuyi Cui

State Key Laboratory of Urban Water Resource and Environment, School of Municipal and Environmental Engineering, Harbin Institute of Technology, Harbin 150090, People’s Republic of China

Abstract

This study aimed at evaluating the properties, nutrients removal and microbial community dynamics of the algal-bacterial granules in a lab-scale photo-bioreactor treating synthetic domestic wastewater. Two identical bioreactors were operated under non-phototrophic (Rc, the control) and phototrophic (Rp) conditions (12h/12h dark-light cycles), respectively. The results showed that the aggregation of algal and bacterial biomass was rapidly formed in Rp, possibly as a nuclei of the granule, which favored the achievement of aerobic granulation. The mature algal-bacterial granules had compact structure, large size and exhibited excellent settleability, besides, more biomass production was facilitated in Rp compared to the control (Rc). Notably, more production of extracellular polymeric substances (EPS) was induced in Rp, especially aromatic and tryptophan protein-like materials, implying that the proteins contributed significantly to the formation and stability of algal-bacterial granules. Further investigations revealed that algal-bacterial granules provided resistance to overcome the temperature fluctuations and enhanced the removal efficiencies of organics and nutrients (nitrogen and phosphorus), probably attributable to the enrichment of the classes Betaproteobacteria, Gammaproteobacteria and Flavobacterium, and the algae classes Chlorophyta, Trebouxiophyceae and Bacillariophyceae. Whereas the Cyanobacteria only occupied a small proportion. These findings demonstrated that the algal-bacterial consortium was conducive to accelerate the aerobic granulation, enhance the system stability and improve the nutrients removal efficiency.

Keywords: algal-bacterial granulation, extracellular polymeric substances (EPS), photo-bioreactor, wastewater treatment

Professional Biography

Bing Zhang is a PhD student from School of Municipal and Environmental Engineering, Harbin Institute of Technology. Her research direction is the wastewater treatment by the aerobic granular sludge (AGS), especially on the formation mechanisms of the algal-bacterial

aerobic granular sludge and the interactions between bacteria and algae based on the microbial quorum-sensing (QS).

Email Id: 412051542@qq.com

Any Comments: