Abstract Title: Performance analysis of an algal-activated sludge symbiotic system on wastewater treatment, algal-activated sludge characteristics and community structure: Effect of sludge concentration

Li Sun, Yu Tian, Jun Zhang,

1, 2, 3 State Key Laboratory of Urban Water Resource and Environment (SKLUWRE), School of Environment, Harbin Institute of Technology, Harbin 150090, China

Abstract

This study focused on the effect of different sludge concentration on the performances of an algal-activated sludge symbiotic system in terms of wastewater treatment, algal-activated sludge characteristics and community structure. The results showed that the highest wastewater treatment performances were obtained in the reactor R2 (sludge concentration of 700 mg/L) with sCOD, NH4+-N and PO43--P removal efficiencies of 90.6±2.3%, 97.69± 2.6% and 83.81±2.3%, respectively. The relative coefficient between Chlorophyll-a (Chl-a) concentration and nutrients removal was in the declining trend of NH4+-N > PO43--P > sCOD, and decreased with the increase of sludge concentration indicating the increased sludge concentration inhibited the growth of algae. Further investigation exhibited that different sludge concentration resulted in the changes of DO and pH influencing the wastewater treatment and algae growth in the symbiotic system. Under the situation of nutrient deficiency, algae displayed superior nutrient utilization ability than sludge bacteria, contributing to the death and disintegration of sludge bacteria which led to the decline of TSS content after day 4. In addition, analysis on the EPS production and Ps/Pr results revealed that the better nutrients removal and algae growth and the preferable settleability in R2 was positively related to its higher EPS production (99.79mg/g-VSS) and Ps/Pr. The DGGE profiles and gene sequences analysis demonstrated that some new species appeared and the functional microorganism was enriched under different sludge concentration, suggesting that both bacteria and algae had a selective power for particular members of each other. This study would provide some novel insights into the relationship between algal-activated sludge and be helpful to develop the algae-activated sludge system.

Keywords - Algal-activated sludge symbiotic system; Sludge concentration; Wastewater treatment; Algal and bacterial communities.

Professional Biography

My name is Li Sun from School of Environment in Harbin Institute of Technology. I am a Ph.D candidate. I am also a member of the State Key Laboratory of Urban Water Resource and Environment. I do enjoy learning things every day, reading, studying have become my habit, concentration on books make me forget the noise from the material world, make me find a kind of inner treasure from human beings. Inside of my heart, I feel there is a responsibility to appreciate the society, by providing health care, clean environment, fair education. As an educator, and a future researcher, I will devote all my time, education and life to return what I get from the others.

Paste your Photograph here

Email Id:sunli_hit@163.com

Any Comments: