Combining innovant electrochemical technology and ozonation process for the treatment and recycling of grey wastewater from industrial laundries

Rimeh Daghrir (Center of Water Technology, Montreal, Canada).

Abstract

In Canada, water pollution has become a serious environmental problem that has harmful consequences on public health. Grey wastewater (GWW) generates a large volume of wastewater containing a very high concentration of organic matter, total suspended solids, oil & grease. Therefore, extensive efforts focus on developing powerful methods for sustainable GWW reuse. The most common application for GWW reuse is toilet flushing, which can reduce water demand within dwellings by up to 30 %. The most stringent criteria  for recycling treated GWW require a biochemical oxygen demand (BOD) of less than 10 mg/L, a turbidity below 2 NTU and a non detectable level of either total or faecal coliforms.

Ozonation is an efficient and powerful oxidizing process. However, an application of ozone alone in wastewater treatment is often limited by the cost of in-situ ozone production and by the high ozone consumption. Nowadays, a great interest in advanced electrochemical technologies has been emerging. Electrochemical treatments are characterized by simple equipment, brief retention times and easy operation. Electrocoagulation (EC) process is an effective technology for removing greases, suspended solids, organic and inorganic pollutants from different types of industrial wastewater. Besides, electro-reduction (ER) is also an effective electrochemical technique able to produce in situ H2O2 (1.77V) by cathodic reduction of oxygen. From this point of view, the aim of the present study is to investigate a hybrid technique combining EC, ER and ozonation processes (called ECRO process) and to evaluate its performance in treating and recycling GWW. Under optimal conditions, the ECRO process reduced more than 95% of CODT, 100% of BOD, 99% of turbidity, 92% of TSS, more than 95% of O&G and 100% of phenolic compounds. The development of this technology at large scale could present an alternative to avoid the drawbacks of conventional water treatment methods.

Keywords: grey wastewater, Electrocoagulation, electro-reduction, ozonation

Professional Biography

Dr Daghrir completed his doctorate in Water Science at the University of Quebec (National Institute for Scientific Research), Canada in 2013. Actually, doctor Daghrir is Researcher at Center of Water technology, CTE (Montreal, Canada). His research activities are focused on the development of innovative technologies for environmental applications, mainly to remove persistent organic, inorganic and microbial pollutants from waters and wastewaters by using electro-technology and oxidative processes. Dr Daghrir is author and co-author of more than 30 research papers published in international and scientific journals, 30 conference presentations, and more than 35 research reports related to environmental technologies.

Recent Publications

Rimeh Daghrir, Abdelaziz Gherrou, Isabelle Noel; Seyhi Brahima (2016) hybrid process combining electrocoagulation,  electroreduction and ozonation processes for the treatment of grey wastewater in batch mode. Journal of Environmental Engineering. 142 (5). DOI: 10.1061/(ASCE)EE.1943-7870.0001071.

Karama Jardak; Patrick Drogui; Rimeh Daghrir (2015) Surfactants in aquatic and terrestrial environment: occurrence, behavior, and treatment processes. Environmental Science and Pollution Research. DOI 10.1007/s11356-015-5803-x.

Nazar Delegan; Rimeh Daghrir; Patrick Drogui; My Ali El Khakani (2014) Bandgap tailoring of in-situ nitrogen-doped TiO2 sputtered films intended for electrophotocatalytic applications under solar light. Journal of Applied Physics. 116 (15), 153510.

Rimeh Daghrir, Patrick Drogui, Seyhi Brahima, Aicha Dimboukou-Mpira (2014) Photosonochemical degradation of Butyl-paraben: optimization, toxicity and kinetic studies. Science of the total Environment. 490:223-234.

Rimeh Daghrir, Patrick Drogui (2014) Efficient treatment of domestic wastewater by electrochemical oxidation process using bored doped diamond anode. Separation and Purification Technology. 131: 79-83.

Email Id: rdaghrir@cteau.com