Flyash based Coating Materials for Cooling Application

Afsha Anjum, Avneesh Anshul, S. S. Waghmare, Sadhana Rayalu

CSIR-National Environmental Engineering Research Institute, Nagpur, 440020, India

*email: anshul@neeri.res.in

The environmental concerns over carbon dioxide emission by cement industries have led to the search of alternative binding cementous materials. In this content an important endeavour of developing fly ash based zeolites, organo zeolites and titania composites have been undertaken at CSIR-National Environment and Engineering Research Institute, India. Materials are being developed for smart building and green structured applications to minimize energy and material consumption and hence reducing pollution. Inorganic aluminosilicate three-dimensional matrix composition with enhanced interfacial binding and homogeneity is achieved with flyash and alkali activator.  The use of industrial waste derived zeolite with super hydrophilicity properties has been developed for coating on building and other green structural applications. The titania photocatalyst with super hydrophilicity as coating material have self-cleaning and air purification properties. The ideal combination of zeolite in geopolymerised fly ash are derived for use as coating material for buildings for heat management based on concept of solar heat pump. The cavities in the SiO4 tetrahedral crystals of aluminium silica frame occupies water molecules behaves as molecular sieves of size 1μm shown in the SEM image for natural cooling behavior. The crystalline phases present in the zeolite fly ash coating material were identified by X-ray diffraction. X-ray diffraction pattern of matrix showed consistency with reports and indicated the presence of quartz, mullite, silicon oxide, silimanite as major crystalline phases. The diffraction pattern also shows amorphous patter of sodium aluminate and sodium silicate as geopolymeric binding gel. Microscopic analysis of the developed coating materials using scanning electron microscope (SEM) show interfaces between concrete and coating material compositions show a uniform morphology and cavities. Image also have well established signature of glassy amorphous phase of geopolymeric gel and unreacted fly ash. The thickness of the coatings was 100 μm to 150 μm. Absorption bands were recorded between 4000-500 cm-1 using Fourier transform infra-red spectrometer. The FTIR spectra developed fly ash-zeolite coating material revealed that a broad band around 1000 cm-1 and 1455 cm-1 in due to Si-O-Si. The shifting of Al-O-Si towards lower wave number is due to asymmetric stretching vibrations and aluminosilicate geopolymeric gel. Fly ash spectra are clearly seen as also present in XRD peaks. The adhesion strength of the coating material on concrete substrate were done using an Elcometer have values 0.4 MPa and 0.38MPa respectively.

Keywords: Geopolymer, Fly ash, Coating

Professional Biography (100-150 words)

Ms. Afsha Anjum, Research Scholar at CSIR-National Environment Engineering

Research Institute has completed her Master degree in Science with specialization

in Chemistry. She did her Master’s thesis from VNIT (Vishveshwarya National Institute of Technology), Nagpur.

Dr. Avneesh Anshul, Ph. D. (Physics), has completed his PhD “the effect of

nano to microscopic disorders on the Electrical and Magnetic Properties of

lanthanum Manganites”. He is currently working at Quick Hire Scientist at

CSIR-National Environment Engineering Research Institute in Environmental Materials Division. He has published more than 23 papers in reputed journals and has 7 patents granted and filed in his credit.

Recent Publications (up to 5)

- A. Anshul (2017); Studies on fly ash based geopolymeric material for coating on mild steel by paint brush technique, 75, 139-144.

- A. Anshul (2017); Large low-field magnetodielectric response in epitaxial thin film of multiferroic Bi2NiMnO6; JPhysD-111195.

- A. Anshul (2017); Nano phosphatic hybrid geopolymeric corrosion resistant coating material and a method of preparation thereof. 20,170,267,870

- A. Anshul (2017); Advanced non-toxic radiation shielding materials from tailored brine sludge and a process for the preparation thereof. US Patent 15/449,088

- A. Anshul (2017); Facile and controlled synthesis of copper sulfide nanostructures of varying morphology Journal of Materials Science: Materials in Electronics, 28,7, 5597-5602.

Email Id: anshul@neeri.res.in