Solid Fuel Production from Non-segregated Municipal Solid Wastes Employing the Hydrothermal Treatment

Kunio Yoshikawa

Tokyo Institute of Technology, Japan

Abstract (not more than 300 words)

Up to now, the only commercialized ways of municipal solid waste (MSW) treatment are mass land-filling and mass burning. In Japan, most of burnable wastes are incinerated, but not in other countries, and still land-filling is the most popular way of waste treatment all over the world. But the world recent trend is to prohibit or limit land-filling of wastes while citizens do not want to increase waste incineration in developed countries as well as developing countries. On the other hand, major part of the world is discharging non-segregated municipal solid wastes. Thus we have to find out the utilization ways alternative to incineration for non-segregated MSW. Pre-treatment of wastes requires crushing, drying and deodorizing, which are normally different processes. But we have developed innovative hydrothermal treatment technology (HTT) which can perform these three pre-treatment functions in one process utilizing high pressure saturated steam. Figure 1 shows the operating principle of HTT. Non-segregated MSW are fed into the reactor, and then, 220℃, 2.5MPa saturated steam is supplied into the reactor for about 30 minutes and the blades installed inside the reactor rotates to mix MSW and steam for about 10 minutes. Then the product is discharged after extracting steam. The product is powder-like substance and the moisture content is almost the same as the raw material, but is easily dried by natural drying. The inert material such as metal, glass and stones can be easily sieved out after drying. There is almost no bad smell in the solid products, and the products can be used as solid fuels which can be easily mixed with coal for power generation or cement production. Only 10-15% of the product is enough for steam production in a boiler. HTT has already commercialized in Japan, China and Indonesia.

Keywords— Alternative to incineration, Hydrothermal treatment, Municipal solid waste, Solid fuel production

Professional Biography (100-150 words)

Prof. Kunio Yoshikawa was born in 1953 in Tokyo, Japan. He is a professor of Tokyo Institute of Technology and an associate editor of Applied Energy. His bachelor, master and doctor degrees were awarded from Tokyo Institute of Technology. His major fields are energy conversion, waste management and environmental engineering. He has been working in Tokyo Institute of Technology for more than 38 years as a research associate, an associate professor and a professor. He has published nearly 200 journal papers with the major award of AIAA Best Paper Award, ASME James Harry Potter Gold Medal, JSME Environmental Technology Achievement Award and Best Educator Award of Tokyo Institute of Technology

Recent Publications (up to 5)

- Tri Sesilia Safril, Bayu Indrawan Safril, Kunio Yoshikawa (2017) Commercial Demonstration of Solid Fuel Production from Municipal Solid Waste Employing the Hydrothermal Treatment. International Journal of Environmental Science 2:316-323.

- Srikandi Novianti, Ilman Nuran Zaini, Anissa Nurdiawati and Kunio Yoshikawa (2016) Low Potassium Content Pellet Production by Hydrothermal-Washing Co-treatment. International Journal of Chemistry and Chemical Engineering Systems 1:28-38.

- Chinnathan Areeprasert, Antonio Coppola, Massimo Urciuolo, Riccardo Chirone, Kunio Yoshikawa and Fabrizio Scala (2015) The effect of hydrothermal treatment on attrition during the fluidized bed combustion of paper sludge. Fuel Processing Technology 140: 57-66.

- Peitao Zhao, Yafei Shen, Shifu Ge and Kunio Yoshikawa (2014) Energy Recycling from Sewage Sludge by Producing Solid Fuel with Hydrothermal Carbonization. Energy Conversion and Management 78: 815-821.

- Pandji Prawisudha, Tomoaki Namioka and Kunio Yoshikawa (2012) Coal alternative fuel production from municipal solid wastes employing hydrothermal treatment. Applied Energy 90: 298-304.

Paste your Photograph here

Email Id: yoshikawa.k.aa@m.titech.ac.jp

Any Comments: None