Abstract Template

Abstract Title: Global warming potential reduction with alternate policy scenarios– the case of a popular tourist site

Usama F. Dawood1, Umaima A. Jalil1, Yasir Adnan1, Izhar H. Shah 1, 2, Hung-suck Park2

1 Institute of Environmental Sciences and Engineering, School of Civil and Environmental Engineering, National University of Sciences and Technology (NUST) H-12 Campus, Islamabad (44000), Pakistan

2 Department of Civil and Environmental Engineering, College of Engineering, University of Ulsan, 102 Dehakro, Ulsan 680-749, Republic of Korea

Abstract (not more than 300 words)

As vehicles are becoming a major source of air pollution worldwide, natural tourist destinations are no exception to this phenomenon as well. With more vehicles and higher concentrations of air pollutants in such areas, the ecological balance is being altered. With the aim of improving the current trend of atmospheric degradation via tourist based vehicles, both the existing emissions and alternate-transportation-mode emissions were calculated using a U.S. Environmental Protection Agency (EPA) based emission model. The study site selected was a popular tourist destination in Pakistan having a good road access as well. This study considered pollutants having a known CO2-equivalent Global Warming Potential (GWP) over a 20 years and 100 years horizon including CO2, N2O, CH4, CO, VOC and NOx. Under the base case, annual emissions were found to be 20 kt for CO2, 0.3 tons for N2O, 37.7 tons for CH4, 2.47 kt for CO, 221.3 tons for VOCs, and 238.3 tons for NOx, with a total GWP of 50.96 kt CO2 eq (20 years) 33.66 kt CO2 eq (100 years). The three alternate scenarios included occupancy rate optimization, Euro II, and Euro IV emission standards implementation respectively. Our analysis showed that 20-year GWP can be lowest under the Euro IV scenario (27.25 kt CO2eq) whereas 100-year GWP can be lowest under the Euro II scenario (21.786 kt CO2 eq). Our results prove a significant contribution of these tourist vehicles in overall emissions which can be mitigated via policy level interventions on vehicular technology up-gradation for improved environmental management.

These instructions give you guidelines for preparing papers for conference. Use this document as a template if you are using Microsoft Word. Otherwise, use this document as an instruction set. Define all symbols used in the abstract. Do not cite references in the abstract. Insert images/graphs if any.

Keywords— Air pollution, Environmental modelling, Global warming, Vehicular emissions

Professional Biography (100-150 words)

Izhar H. Shah has completed his MS degree from National University of Sciences and Technology (Pakistan) and currently pursuing doctoral studies from University of Ulsan, Republic of Korea. He currently is enrolled in the Department of Civil and Environmental Engineering, and conducts his research work at the Ecosystems Laboratory. He has published 3 papers in reputed journals and has been serving as a faculty member (on study leave) at National University of Sciences and Technology (Pakistan).

Recent Publications (up to 5)

- Izhar Hussain Shah and Muhammad Zeeshan, 2016. Estimation of Light duty vehicle emissions in Islamabad and climate co-benefits of improved emission standards implementation. Atmospheric Environment 127, 236-243.

- Izhar Hussain Shah, Ghulam M., Raza H. , Zameer A., Khalid H., Amin Q., Amber S.,  Panhwar F., and Memon F., 2016. Analysis of Rush Hour Driving Conditions and Vehicle Specific Power Distribution in a Large Urban Area: Case study of a Major Highway. Academia Journal of Environmental Sciences 4(6), 105-110.

- Izhar Hussain Shah, Ghulam M., Raza H. , Zameer A., Khalid H., Amin Q. and Amber S.,  2015. Prediction of running emission factors from passenger vehicles in an urban area using International Vehicle Emissions Model as a computational tool. Journal of Multidisciplinary Scientific Research 3(5), 17-21.

Paste your Photograph here

Email Id:

izhar88@hotmail.com

Any Comments:

None