Application of response surface methodology for optimization of lead bioaccumulation parameters in Gammarus pulex (L., 1758)

1Ince M, 2Onal A, 3Pala A, 3Serdar O

1Faculty of Engineering, Department of Chemical Engineering (Munzur University, Turkey) 2Faculty of Engineering, Department of Food Engineering (Munzur University, Turkey)

3Faculty of Fisheries (Munzur University, Turkey)

Abstract

The aquatic system pollution is an important environmental problem. Accumulation of high concentrations of contaminants in marine zones result from human's activities. Industrial contaminants especially heavy metals receive considerable attention from scientists and engineers because of their toxic potential and pose a serial risk for human health when they enter the food chain and water bodies. As heavy metals are not decomposed in environment, their concentrations should be reduced to acceptable levels before disposed into the environment. Lead is a priority hazardous element even at trace levels; it spread into the aquatic environment by various industrial processes. Their main impact on aquatic environment is the poisoning of aquatic organisms and severe changes in the aquatic fauna and flora. The characteristic symptoms of chronic Pb toxicity lead to several health problems including the liver, kidneys and heart, and immune system disorders.

The aim of this research was to evaluate the suitability of Gammarus pulex (L., 1758) as a biomonitor for lead (Pb) and its’ bioaccumulation conditions were optimized using Box-Behnken design whether it can be used as a predictive tool to assess aquatic quality. A Box–Behnken experimental approach combined with response surface modeling was employed for maximizing Pb2+ bioaccumulation in Gammarus pulex (L., 1758) based on various experimental data. Independent variables significance and their interactions were tested using analysis of variance (ANOVA). The standardized effects of the independent variables and their interactions on the dependent variable were also investigated. Optimum experimental conditions were obtained as temperature; 22.0 ºC, contact time; 8 hours, metal concentration; 69.5 µg L-1. Based on Box-Behnken design, applied model is significant because model F-value and R2 were obtained as 77.80, 0.9901, respectively.  Bioaccumulation of Pb amount was analyzed using electrothermal atomic absorption spectrophotometer. The obtained data suggest that G. pulex can be evaluated as Pb biomonitor.

Keywords—Bioaccumulation, Box-Behken design, Gammarus pulex (L., 1758), Lead

Professional Biography

Dr. Muharrem INCE received his Ph.D. degree in analytical chemistry at Firat University, Turkey in 2008.  He worked as a research analytical chemist in Science Education Department at Mus Alparslan University, Turkey from 2009 to 2012 as assistant professor. He has been working at Munzur University since 2012. From 2013-2016, he served as a Head of Department of Chemical Engineering at the Munzur University, Turkey. He becomes as an Editorial Board member of the International Journal of Food and Nutrition Sciences, International Journal of Toxicology and Environmental Health, Journal of Environment and Waste Management, International Journal of Pure and Applied Sciences, International Research Journal of Chemistry and Chemical Sciences and Science Journal of Analytical Chemistry. Currently, He is an Associate professor at Munzur University, Turkey. He is an author and co-author of more than 22 papers that published in journals with good impact factors.

Recent Publications

- Ince M, Kaplan Ince O (2017) Box–Behnken Design Approach for Optimizing Removal of Copper from Wastewater Using a Novel and Green Adsorbent, Atomic Spectroscopy, 38(6), 200-207.

- Ince M, Kaplan Ince O, Asam E and Önal A (2017) Using food wastes biomass as effective adsorbents in water and wastewater treatment for Cu(II) removal, Atomic Spectroscopy,  38(5), 142-148.

- Ince M, Kaplan Ince O, Yaman M (2017) Optimization of an Analytical Method for Determination of Pyrene in Smoked Meat Products, Food Analytical Methods, 10(6),  2060–2067.

- Kaplan Ince O, Ince M, Yonten V and Goksu A (2017) A Food Waste Utilization Study for Removing Lead(II) from Drinks, Food Chemistry, 2017 (214)  637–643.

- Yonten V, Ince M, Tanyol M, Yildirim N (2016) Adsorption of Bisphenol A from aqueous solutions by Pleurotus eryngii immobilized on Amberlite XAD-4 using as a new adsorbent, Desalination and Water Treatment, 57 (2016) 22362–22369.

Email Id: muharremince@munzur.edu.tr

Any Comments: