Smart glasses in warehouse – an ergonomic view

Natasa Vujica Herzog1, Borut Buchmeister1, Amer Beharic2, Brigita Gajsek3

1 University of Maribor, Faculty of Mechanical Engineering, Maribor, Slovenia

2 Healthcare Center dr. Adolfa Drolca Maribor, Maribor, Slovenia

3 University of Maribor, Faculty of Logistics, Maribor, Slovenia

Abstract

Smart glasses or data glasses presents a novel technological equipment with a great potential in different environments and especially in warehouses. Despite of several benefits such as supporting workers with right information at right time, improved productivity because of both free hands and others, there are also possible problems evidenced from literature that work with Head-mounted displays (HMD) can cause headaches, pressure in eyes, problems with focusing and difficulties with text reading. To study addressed problems we performed a research together with ophthalmologists from Maribor Healthcare Centre. The research was funded by The Public Scholarship, Development, Disability and Maintenance Fund of the Republic of Slovenia. We tested effects of using smart glasses Vuzix M300 during order picking activities that last four hours in testing warehouse environment at the Faculty of Mechanical Engineering. We tested users comfort during wearing smart glasses through detailed questionnaire, performed several eyes investigations before and after use of smart glasses and performed several ergonomics analyses about workers postures during work. Results confirmed some benefits but also obstacles. Among them, two ergonomics problems can be exposed: 1) tested HMD is very heavy and not the most appropriate for long-term use, and 2) since start button for scan function is on smart glasses a constant posture with lifted right arm is present that can be harmful for worker according to ergonomics standards.

Keywords — smart glasses, warehouse, ergonomic analyses, survey, questionnaire

Professional Biography

Natasa Vujica Herzog is an Associate Professor at the Faculty of Mechanical Engineering, University of Maribor, Slovenia. Her research area is operations and production management in particularly ergonomics. She is author or co-author of more than 100 publications, several original scientific articles published in international journals, monographs, and chapters in scientific books.

Borut Buchmeister is a Professor at the Faculty of Mechanical Engineering (University of Maribor, Slovenia). He is Head of Laboratory for Production & Operations Management and Laboratory for Discrete System Simulation. Since 2002, he is the Editor-in-Chief of the International Journal of Simulation Modelling.

Amer Beharic is an ophthalmologist specialist in Healthcare Center at the Unit of Occupational Medicine, Zdravstveni dom dr. Adolfa Drolca Maribor. Before that, he worked as a specialist for Strabsimology subspeciality covering surgical and conservative therapy. He is author and co-author of some scientific articles in international journals.

Brigita Gajsek is an Assistant Professor at the Faculty of Logistics, University of Maribor, Slovenia. Her research interest includes processes, organization, technologies in intra-logistic and project management. She is Vice President of Slovenian Association for Project Management and editor-in-chief at regional scientific journal Projektna Mreža Slovenije.

Recent Publications

- Gajšek B, Vujica Herzog N, Butlewski M, Đukić G (2017) Research opportunity: incorporation of human factors in order picking system models. Zeszyty Naukowe Politechniki Poznańskiej : Organizacja i Zarządzanie, 72:45-61.

- Vujica Herzog N, Zavec Pavlinič D, Kuzmanović BD, Buchmeister B (2016) Thermal manikin and its stability for accurate and repeatable measurements. International journal of simulation modelling, 15:676-687,

- Šauperl O, Vujica Herzog N, (2014) Workplace design in the clothing shop by considering anthropometry. Asian academic research journal of multidisciplinary, 1:107-122.

- Vujica Herzog N, Vujica Beharić R, Beharić A, Buchmeister B (2014). Ergonomic analysis of ophthalmic nurse workplace using 3D simulation. International journal of simulation modelling, 13: 409-418.

Natasa Vujica Herzog

Borut Buchmeister

Amer Beharic

Brigita Gajsek

Email: natasa.vujica@um.si, borut.buchmeister@um.si, Amer.Beharic@gmx.net, brigita.gajsek@um.si