Abstract Template

Design of a novel polarization dependent 1x2 optical switch and a 2x1 MUX/DEMUX based on polarization controllers, polarization beam splitters and combiners

F. KARIM

1 Department of Telecommunications Faulty of Technology Chetouane (Aboubekr Belkaid University of Tlemcen, ALGERIA)

Abstract (not more than 300 words)

In this paper, we present a 1x2 electro-optical switch based on a 1x2 polarization beam splitter (PBS), a 2x2 polarization beam combiner (PBC) and two TE/TM polarization converters. By applying a voltage of 0/+5 V simultaneously on the two converters, an optical signal is routed to the cross port or to the bar port. We have used the light polarization principle to eliminate the polarization dependent loss. The total insertion loss obtained through simulation is 0.6 dB, while the switching time is estimated to 20 μs. The estimated extinction ratio of this switch is 30 dB. We present also on this paper, an all-optical 2x1 Multiplexer based also on PBSs, PBCs and a polarization controller. The insertion loss found by simulation is 0.5 dB. The demultiplexing function is done by placing only a 1x2 polarization beam splitter after this multiplexer. The two proposed devices operate on the 1550 nm wavelength with a 40 nm pass band.

Keywords—optical switch; multiplexing, demultiplexing; beam splitter; beam combiner; polarization controller.

Professional Biography (100-150 words)

Fethallah Karim has completed his PhD at the age of 28 years from Aboubekr Belkaid University of Tlemcen. He acts actually as an assistant professor at the Faculty of Technology of Tlemcen. He is an active researcher with the Photonics research team at the Laboratory of Telecommunication of Tlemcen. His research interests include optical communications, optical filters synthesis, electro-optical switches and multiplexers design. He has published several papers in reputed journals and has been serving as a  reviewer for several indexed conferences and journals.

Paste your Photograph here

Email Id:  karim.fethallah@gmail.com

Contact Number: 00213542 98 39 86

Any Comments:

Iam searching a sponsor, an external laboratory or an industrial company to make the realization  and the test of the two proposed devices presented in the abstract above. Please promote my request to the concerned staff.