ABSTRACT

Impact assessment of hydrological patterns on Porto-Novo lagoon water, sediments quality characteristics, and pollution status were carried out for 12 months. Water quality parameters and metal pollutants were also evaluated. The water body was stratified into 12 strata based on the morphology of the lagoon and various human activities on the lagoon. Data were collected based on season, sampling stations, metal pollutants, and water quality parameters. The results were analyzed in the laboratory and interpreted using exploratory data analysis statistics. There is no significant difference in the means of dissolved oxygen and pH in different months between July and June at 5% level of probability. However, there is a significant difference in the means of temperature, turbidity, electrical conductivity,  salinity, total dissolved solids, COD, BOD, total hardness, and depth in different months at 5% level of probability. The order of amount of metallic contaminants in sediments from Porto-Novo Lagoon between July and December (Rainy season) are in these order; Mn˃ Fe˃ Zn˃ Pd˃ Cr˃ Cu˃ Ni˃ Cd˃ Va˃ Hg˃ MH3Hg. The metallic pollutants are significantly difference (p=0.0) during rainy season. The trends of existence of metallic pollutants in sediments from Porto-Novo Lagoon from January to June (dry season) are in these order; Fe˃ Mn˃ Zn˃ Cu˃ Cr˃ Pd˃ Ni˃ Cd˃ Va˃ Hg˃ MH3Hg. The difference of the metal pollutants in the dry season at p=0.05 is significance. It is obvious that the hydrological periods affected all the tested physicochemical characteristics and also determine the metal pollutants status of Porto-Novo Lagoon water and sediments. It is therefore recommended that the state of emergency should be declared on Porto-Novo Lagoon for adequate environmental revamping and protection by environmental expert and regulatory agencies.

Keyword: Pollution, Hydrological Pattern, Sediments, Water, Bionetwork