Abstract

Abstract Title: Decorating TiO2 nanotubes with C3N4 for photocatalytic removal of organic pollutants and anaerobic digestion of sludge

Muzammil Anjum1, Rajeev Kumar1, S.M. Abdelbasir2, M.A. Barakat1,2*

1Department of Environmental Sciences, Faculty of Meteorology, Environment and Arid Land Agriculture, King Abdulaziz University, Jeddah, Saudi Arabia

2Central Metallurgical R & D Institute, Helwan 11421, Cairo, Egypt

*Corresponding Author’s Email: mabarakat@gmail.com

Abstract

Herein, C3N4 and TiO2 NTs were considered for the synthesis of visible light active C3N4/TiO2 NTs composites (with different melamine concentrations) by high temperature calcination method. The co-existence of C3N4 and TiO2 NT and visible light activity was confirmed by XRD, TEM, UV-visible and PL spectroscopy. The photocatalytic performance of TiO2 NT with 2 % of melamine (precursor of C3N4), enhanced the degradation of 2-chlorophenol (k = 0.0176 min-1), where 96.6 % removal was achieved at optimum pH 7.0 and pollutant load of 30 mg/L. The application of C3N4/TiO2 NTs for solublization of rigid structure of sludge by photocatalysis released the soluble organics showing an improvement in sCOD production (4587 mg/L). Subsequently, anaerobic digestion of solubilized sludge has improved the methane production (723.4 ml kg-1 VS) by 1.37 and 1.6 times compared to that in anaerobic digestion with photolytic and raw sludge, thus showing a promising applicability for biogas production from sludge.

Keywords—TiO2 semiconductors, photocatalysis, pollutants, wastewater sludge

Professional Biography (100-150 words)

M.A. Barakat has completed his PhD at the age of 30 years from Cairo University, Egypt, and postdoctoral studies from University of Delaware, USA. Currently, he is Prof. of Environmental sciences, King Abdulaziz University, Saudi Arabia. He has published more than 100 papers in reputed journals and has been serving as an editorial board member of repute.

Prof. Barakat is highly qualified in the field of Industrial Waste Management and Pollution Control, besides Catalysis and Nanotechnology. Experience includes academic research and teaching works in Japan, Germany, the United States, and Saudi Arabia, as well as initiating and leading industrial research projects in Egypt, and jointly with the United states of America.

- Research ID: H-7499-2012,

http://www.researcherid.com/rid/H-7499-2012

- Scoups I D: Barakat, M. A. (7101803609)

https://www.scopus.com/authid/detail.uri?authorId=7101803609

Scoups citation: 2482,    Highly cited 593,       H index: 22

Email Id: mabarakat@gmail.com

Any Comments:

I can present also a key note talk in that field if the Conference can support all the expenses of travelling.