# Application of Artificial Neural Networks for the prediction of performance of Desiccant Material for Desiccant Cooling System

Hemant Parmar, Ph.D.

Ujjain Engineering College, Ujjain (M.P.)- India

### Abstract (300 word limit)

### Desiccant cooling system (DCS) is an alternate suitable option against conventional cooling system in humid climates. A typical system combines a dehumidifier that uses dry desiccant wheel with desiccant material, with direct or indirect evaporative systems and a sensible cooling system. DCS is the environmental protection technique for cooling purpose of the building. Desiccant wheel is main key component of the DCS and this wheel has desiccant material. The desiccant materials may be a suitable option for better use of evaporative cooling techniques in warm and humid climate. The dehumidification of air by adsorption is a physical process by desiccant materials that attracts the molecules of water present in air on the adsorbent surface. Desiccant materials attract moisture from the air by creating an area of low vapour pressure at the surface of desiccant. Some common adsorbents used are SiO2 (Silica gel), LiCl, Al2O3 (Activated alumina), LiBr and Zeolite. This system reduces the level of chlorofluorocarbons (CFC) in the environment because it restricts the use of conventional refrigerant. An Artificial Neural Network (ANN) was constructed to predict symptoms of desiccant wheel for any climatic conditions. The symptoms of desiccant wheel were outlet specific humidity and outlet dry bulb temperatures. Feed-forward network was employed with resilient back-propagation (trainrp) algorithm used as the training function. The outputs from the network were obtained and validated with the experimental results. Among the constructed networks, the best prediction performance was observed in two-hidden-layered network with minimum error. The modelling of the desiccant wheel was carried out with neural network toolbox of MATLAB® with two inputs (specific humidity and temperature at inlet) and two outputs (specific humidity and temperature at outlet) values. The performance index (Mean square error) for specific humidity and temperature were calculated as 0.00209 and 0.00579 respectively. The proposed model may be used for any climatic conditions to predict the output from the desiccant wheel for the design of the solid desiccant based cooling system in humid climate.

Key words: Desiccant Cooling system, Desiccant material, Desiccant wheel, Neural network, Modelling, Training algorithm.

### Recent Publications (minimum 5)

- Parmar H., Hindoliya D.A., 2013, “ Performance of solid desiccant based evaporating cooling system under the climatic zones of India”. International Journal of Low Carbon Technologies, vol.8, pp 52-57.

- Parmar H., Hindoliya D.A., 2011, “ Artificial Neural Network based modeling of Desiccant Wheel”. Energy and Buildings, vol.43, pp 3505-3513.

- Parmar H., Hindoliya D.A., 2011, “ Solid Desiccant Cooling System Employed with Ventilation Cycle”. Journal of Institution of Engineers (Springer) , DOI 10.1007/s40032-012-0038-9.

- Parmar H., Hindoliya D.A., 2011, “ Desiccant Cooling System for Thermal Comfort: A Review”. International Journal of Engineering Science and Technology, vol.3, pp 4218-4227.

- Parmar H., Hindoliya D.A., 2018, “ Studies on Job Satisfaction Level Among Employees of Dairy Based Industry”. Journal of Industrial Engineering, Accepted, In Press.

Biography

Hemant Parmar has expertise in the field of Desiccant Material and Desiccant Cooling System for thermal comfort. He is also an expert in the field of Energy and Environment. He has published many research paper in different areas. His work on the Desiccant Cooling System creates new pathways for protection of the environment to reduce CFC level.

Presenting author details

Full name (As per the passport): Hemant Parmar

University/Industry/Organization name: Ujjain Engineering College, Ujjain (M.P.)- India

Designation: (Director/Prof/Assistant or Associate prof/Researcher/Student): Assistant Professor

Country: India

Mobile number (country code – number) :+919425988231

Email: hemant_parmar1@rediffmail.com
Category (Oral presentation/ Poster presentation/Young researcher forum/E-poster): Oral presentation

Research interest: Desiccant Materials

Your profile page URL in your department:

Year of birth: 1976

Linked In/Facebook/twitter account:

Note: Please fill up the Red mark fields so that it would be easier for us to cite your details in the conference particulars. (Please do not forget to fix your photo and Logo above the title)