# Biodegradation  of  cotton stalks  for  protein-rich  feed  production by                                                       co-cultivation of Pleurotus ostreatus and Azospirillum brasilense grown               on  solid state fermentation

# Bahaa T Shawky

Microbial Chemistry Department, Genetic Engineering and Biotechnology Research Division, National Research Centre, 33 El Bohouth Street, Dokki,   Giza, 12622, Egypt

Abstract

The alarming rate of population growth has increased the demand for food production  leading to a yawning gap in demand and supply. The rapid growth, high protein content of microbes and their ability to utilize  waste feed stocks  have made microorganisms prime candidates for use as animal feed protein supplement. Crop residues  are generated  in huge quantities  every year,  creating disposal problems in the absence of its proper utilization. This study was related to the potential of co-cultivation of Pleurotus ostreatus and Azospirellum brasilense, evaluation of cotton stalks waste, and also a contribution to prevent environmental pollution. Solid state fermentation (SSF) offer advantages  such as biodegradation of crop residues for  production of  protein-rich feed. The potential of  P. ostreatus, a white-rot fungus  and  A. brasilense a nitrogen-fixing bacterium  co-cultivation was assessed for the degradation of chopped cotton stalks . SSF was carried out for 5, 10, and 15 days at 30 °C.  Samples of the fermented cotton stalks were analyzed for crude protein, cellulose, hemicellulose, lignin, and in vitro dry matter digestibility. Activities of ligninolytic enzymes were determined. The  lignin, hemicellulose, and cellulose in untreated  substrate was 23, 27, and 40 %, respectively. Co-cultivation  resulted in the degradation of 20 % lignin, 15 % hemicellulose, and 17 % cellulose.  P. ostreatus degrades lignocellulose with obvious selectivity for lignin, resulting in lignin content significantly decreased and in vitro digestibility was increased The degradation by-products of  P. ostreatus was metabolized by  A. brasilense, leading to more extensive synergistic use of lignocellulosics, which reflects  on the enhancement of lignin degradation  as well as increased laccase and peroxidase activities.  The crude protein content of  the fermented cotton stalks  was 15 %. Extensive studies of single cell protein production for bridging the gap between demand and supply is a major step in this direction.

Recent Publications:

- Bacha U, Nasir M, Khalique A, Anjum AA, Jabbar MA (2011). Comparative assessment of various agro-industrial wastes for Saccharomyces cerevisiae biomass production and its quality production: as a single cell protein. J. Anim. Plant Sci. 21: 844 849.

- Bahaa TS    (2017)  Conversion of rice straw to fermentable sugars and bioethanol by MFEX  pretreatment and sequential fermentation        MATTER: International Journal of Science and Technology.  3 : 356-380

- Gour S , Mathur N , Singh A (2015)   Single Cell Protein Production: A Review   Int.J.Curr.Microbiol.App.Sci  4: 251-262

- Matassa S, Boon N, Pikaarand L, Verstraete W (2016) Microbial protein: future sustainable food supply route with low environmental footprint.    Microbial Biotechnology  9: 568-575

- Srividya AR, Vishnuvarthan VJ, Murugappan M, Dahake PG (2014) Single cell protein - a review. Int J Pharm Res Scholars 2: 472-485.

- Suman G, Nupur M, Anuradha S, Pradeep B (2015) Single cell protein production: a review. Int J Curr Microbiol Appl Sci 4: 251-262.

- Tanveer   A (2010)  Production of single cell protein from Saccharomyces cerevisiae by utilizing fruit wastes.  J. Environ. Issues 1: 127 132.

8.	Tesfaw A, Assefa F (2014)  Co-culture: a great promising method in

single cell protein production.  Biotechnol Mol Biol Rev 9: 12-20.

Biography

Professor Bahaa Shawky, His fields of expertise are:  Fermentation Technology and Industrial Microbiology. He acted as Principal Investigator: [1] Reanewable Energy and Environment [2]   Bioprotein [3] Biopolymers [4] Biofertilizers [5] Microbial Enzymes. [6] Plant-Microbe Interactions. Prof. Shawky  has wide range of consultations. He has published more than 100 papers in peer reviewed journals and has been serving as an editorial board member. The holder of an Energy Patent .  The holder of the Invention and Innovation Development Award in the field of Renewable and New Energy. He creates MFEX Prototype, which is a methodology that utilizes lignocellulosic biomass for producing bioethanol. He has built this model after years of experience in research and evaluation. This approach is responsive to all stakeholders. He granted postdoctoral fellowship to: (a) Martin-Luther-Universität (b) Akademie der Wissenchaften, and (c) Humboldt-Universität, Germany. Prof. Shawky worked as Visiting Fulbright Professor at  Texas A&M University, USA.

Email:  btshawky44@yahoo.co.in

Notes/Comments:          Oral Presentation