Abstract Title: Synthesis of nanostructured Co3O4 and Ru doped Co3O4 and their applicability in LPG sensing

Prabhat Kumar Singh1, Neetu Singh1 and  Poonam Tandon1

1 University of Lucknow, Lucknow-India

Abstract

Cobalt oxide and ruthenium doped cobalt oxide were successfully synthesized following the sol-gel route. Their, structural, optical, morphological and elemental studies were performed utilizing number of characterization techniques viz. XRD (X-ray diffraction) technique, FTIR (Fourier transform infrared) spectroscopy, SEM (scanning electron microscopy) and EDS (energy dispersive spectroscopy). The crystallite size of Co3O4 and Ru-Co3O4 nanostructures were calculated to be 38 and 27 nm respectively. Further, a part of the synthesized materials was used for LPG sensing investigations. It was seen that the LPG sensing characteristics were enhanced with the doping of ruthenium in cobalt oxide.  The response time for Co3O4  and Ru doped Co3O4 were calculated to be 35s and 21s respectively. The calculated recovery time for Co3O4 was 66s while for Ru doped Co3O4 was 50s. Thus, the doped material  resulted in lowering of the response and the recovery time. Hence, Ru-Co3O4  can prove to be a prominent material for LPG sensing.

Keywords—Nanomaterials, FTIR, XRD, LPG sensing

Professional Biography

Mr. Prabhat Kumar Singh has received his B.Sc. and M.Sc. Degree in 2010 and 2012 respectively from University of Lucknow, Lucknow, India. Now, he is a UGC-BSR fellow and doing his Ph.D. under the supervision of Professor Poonam Tandon, Department of Physics, University of Lucknow. Currently he is working on “Synthesis and Characterizations of nanostrucrured metal oxides and their Applications as LPG sensor. He has published two papers.

Recent Publications (up to 5)

- Prabhat Singh, Neetu Singh, Satyendra Singh, Mridula Singh, Poonam Tandon, " Synthesis of ZnO nanostructures via sol-gel method and their nanostructural and optical characterization" has been published in the  e-book of ICOPVS-2016 proceedings, ISBN-978-93-5267-364-3

- Neetu Singh, Prabhat Kumar Singh, Anuradha Shukla, Satyendra Singh and Poonam Tandon, "Synthesis and Characterization of Nanostructured Magnesium Oxide: Insight from Solid-State Density Functional Theory Calculations, Journal of Inorganic and Organometallic Polymers and Materials,Vol. 26, pp-1413-1420, 2016, ISSN- 1574-1443, Springer, Impact Factor: 1.508

Email Id:myselfprabhatsingh@gmail.com

Presenting author details

Full name: Prabhat Kumar Singh

Contact number: +919044055861

Session name/ number: Nano Devices and structured materials (Track 3)

Category: (Oral presentation)