# Title: Facile preparation of oxidation resistant magnetite-based nanocomposites for water treatments

Ping Furlan*, Adam Fisher, Alexander Furlan, Monica Keeley, Michael Melcer, David Shinn, and John Warren

U.S. Merchant Marine Academy, Kings Point, New York, USA

* furlanp@usmma.edu

Abstract (300 word limit):

Magnetite nanoparticles (MNPs) - based nanocomposites are promising for drinking water and ballast water treatments due to their easy synthesis and magnetic recyclability.  The bifunctional nanocomposite we recently prepared by incorporating both MNPs and silver nanoparticles into activated carbon matrix has demonstrated recoverability and reusability with high antimicrobial efficiency for water disinfection.  However, prolonged exposure of MPNs to the dissolved oxygen in water converts iron (II) ions to iron (III) ions, weakening the magnetic responsiveness and reducing the recovery rates for the nanocomposites.  In this work, we explore various strategies inhibiting the access of oxygen to the MNPs and preventing them from being oxidized in water.  This includes capping the MPNs with inorganic, organic, and surfactant agents.  The effectiveness is evaluated based on their oxidation resistance in water environments of a wide pH range of 5 – 10.  The nanocomposites with protected MPNs are evaluated for their recoverability and ability to remove water pollutants and/or disinfect water.

Recent Publications (minimum 5)

- P.Y. Furlan, A.J. Fisher, A.Y. Furlan, M.E. Melcer, D.W. Shinn, and J.B. Warren. 2017. Magnetically Recoverable and Reusable Antimicrobial Nanocomposite Based on Activated Carbon, Magnetite Nanoparticles, and Silver Nanoparticles for Water Disinfection. Inventions 2, 10; doi:10.3390/inventions2020010

- P.Y. Furlan, A.J. Fisher, M.E. Melcer, A.Y. Furlan, and J.B. Warren. 2017. Preparing and Testing a Magnetic Antimicrobial Silver Nanocomposite for Water Disinfection to Gain Experience at the Nanochemistry-Microbiology Interface. J. Chem. Educ 94(4), 488-493.

- P.Y. Furlan, Brian Ackerman, Mike Melcer and Sergio Perez. 2016. Reusable Magnetic NanoComposite Sponges for Removing Oil from Water Discharges. Journal of Ship Production and Design 33(4), DOI: https://doi.org/10.5957/JSPD.32.4.160017

- S. Perez, P. Furlan, S. Ellenberger, P. Banker. 2016. Estimating Diluted Bitumen Entrained by Suspended Sediments in River Rapids using O2 Absorption Rate. Int. J. Environ. Sci. Technol. 13(2), 403-412.

- P.Y. Furlan, M. Melcer. 2014. Removal of Organic Water Pollutant Surrogate by Recyclable Magnetite-Activated Carbon NanoComposite: An Experiment for General Chemistry. J. Chem. Educ 91(11), 1966-1970.

- P.Y. Furlan. 2014. Integrating Nanoscience and Technology in Natural Science Associate Degree and Relocation Programs Serving Underrepresented Students. J. Nano Educ 6, 12-24.

- P.Y. Furlan, M.N. Whitney. 2014. Second-generation Stamps for Nanoscale Soft lithography Fabricated by Micromolding in Capillaries. Nano Commun. 1, 52-57.

- .