# Title: Synthesis, Characterization of Cr-Gd Nanocomposites doped with Yttrium Possessing Electrical and Electronic properties

Department of Applied Basic Sciences, Jain College of Engineering and Technology, Hubballi, Karnataka, India-580031

Dr. Vinayak Adimule1* Dr. Adarsh KS2, Dr. Prashanth Banakar 3 ,Miss Tejaswini K Mahendrakar4

Abstract:

Objective: We report here, a set of Perovskite oxide nanomaterials that has been prepared through co precipitation methods. Synthesis of nanocomposite material, chromium-gadolinium doped with various concentrations of yttrium was investigated for their electrical and electronic properties. Method: Nanocompositional hybrid materials were prepared by    co precipitation method with composition of Cr0.3 Gd0.3 (Y2O3) x moles, the gelated precipitate of hybrid nanomaterials were calcinated at different temperatures and  characterized by XRD, FT-IR, UV-Vis, SEM spectroscopic techniques. The doped Y2O3 to chromium and Gadolinium multicomponent nanocomposite with different concentrations     (1-5%). The dielectric constants of the pelletalized samples were examined at room temperature. The precursor material used for the synthesis is chromium sulphate and gadolinium carbonate which are reduced in presence of triethanol amine (TEA) .The aqueous mixture of greenish brown precipitate was filtered, washed with ethanol-water mixture       (1:9 volume %) to remove any impurities present with the precipitate, dried at 50-100°C, heat treated at 650-750°C and obtained pure nanocomposite. Findings: Spectroscopic studies showed that Y2O3 grain size was 30-60nm in diameter and it’s over layer is 85% optically transparent.XRD pattern demonstrated the formation of hexagonal yttrium and dopant addition moderately affect the crystal structure. Higher dielectric constants were obtained with larger grain size of the nanocomposites.

Image

Figure 1: Showing scanning electron microscope image of chromium and gadolinium nanocomposites, doped with yttrium nanoparticles having a particle size of 20nm

Recent Publications

- B. R. Saunders, M. L. Turner, Nanoparticle Polymer Photovoltaic Cells. 2008. Advanced Colloidal  Interface  Science.  138 :1–23.

- Y.F Zhou, M. Eck, M. Kruger, Bulk-heterojunction hybrid solar cells based on colloidal nanocrystals and conjugated polymers 2010. Energy Environmental Science. 3 ;1851–1864.

- M. Wright, A Uddin, Organic-inorganic hybrid solar cells: A comparative review. Solar Energy Mater.2012, Solar Cells 107 ; 87–111

- C Silva, Organic Photovoltaics: Some like it hot. 2013, Nature Mater. 12 ; 5–6.

- . C Deibel, T. Strobel, V. Dyakonov, Role of the charge transfer state in organic donor-acceptor solar cells, 2010, Advanced Materials 22 ; 4097–4111.

Biography

Presently pursuing BE in Computer Science and Engineering and working as Research Student in the area of Nanocomposite, Attended more than 5 National and international conference, workshops, published two articles on doped hybrid nanomaterials in national and international journal. Presently working in developing Transistor and semiconductor diode characteristics of novel compositional nanomaterials under the guidance of Dr. Vinayak Adimule who is working as HOD and Assistant Professor of chemistry at Jain College of engineering and technology, Hubbballi, Karnataka, India

E mail: tkmahendrakar98@gmail.com

NOTES/ COMMENTS: