# Title: The Use of Electron Spin Resonance in Studying the Synergy Be-tween Organic and Inorganic Component in Hybrid Elastomer Based Composites

Aleksandra Ivanoska-Dacikj, Ph D, Research Centre for Environment and Materials, Macedonian Academy of Sciences and Arts, Macedonia.

Gordana Bogoeva-Gaceva, Prof., Faculty of Technology and Metallurgy, Ss. Cyril and Methodius University, Macedonia.

Srečko Valić, Prof., Department of Chemistry and Biochemistry, University of Rijeka, Croatia.

Abstract (300 word limit):

Recently an increasing interest is becoming evident for incorporation of hybrid systems based on different fillers in elastomeric matrices. Multi-walled carbon nanotubes (MWCNT) and organo modified montmorillonite (OMt), two nanofillers different by nature but with demonstrated abilities to enhance the mechanical properties of elastomers were combined in a hybrid filler system and its influence on the nanaocomposites behavior was investigated. Interestingly, the combination of these two nanofillers engenders synergism which results in their better dispersion in different solvents but also in rubber, leading to superior mechanical properties. Here, electron spin resonance (ESR) spectroscopy, which is a powerful technique in the investigation of carbon-based materials, was used for a better understanding of the interactions between the nanofillers. ESR spectra measured by the MWCNT present in the nanocomposites were investigated and the dependence of the double integral of resonance spectra on the EOMt content was established. The temperature dependence of resonance line parameters (resonance line position, width, and double integral) in the range 160–353 K has been also analyzed.

Image

Figure 1: The ESR spectra of 6 phr MWCNT in NR-based nanocomposites containing different quantities of EOMt, at room temperature. Lorentzian shapes of the resonance spectra prove the good dispersion of the nanotubes within the NR matrix, regardless of the pres-ence of EOMt

Recent Publications (minimum 5)

- Ivanoska-Dacikj A, Bogoeva-Gaceva G, Buzarovska A. 2015.  Clay improved dispersion of carbon nanotubes in different solvents. Contributions, Section of Natural, Mathematical and Biotechnical Sciences. 36(1): 5-10.

- Ivanoska-Dacikj A, Bogoeva-Gaceva G, Rooj S, Heinrich G, Wießner S. 2015. Fine tuning of the dynamic mechanical properties of natural rubber/carbon nanotube nanocomposites by organically modified montmorillonite: A first step in obtaining high-performance damping material suitable for seismic application. Applied Clay Science. 118: 99-106.

- Ivanoska-Dacikj A, Bogoeva-Gaceva G, Wießner S, Heinrich G. 2016. Rheometric and dynamic mechanical analysis of complex natural rubber based composites. Contributions, Section of Natural, Mathematical and Biotechnical Sciences. 37: 5-14.

- Ivanoska-Dacikj A, Bogoeva-Gaceva G, Jurk R, Wießner S, Heinrich G. 2017. Assessment of the dynamic behavior of a new generation of complex natural rubber based systems intended for seismic base isolation. Journal of Elastomers and Plastics. 49: 595-608.

- Ivanoska-Dacikj A, Bogoeva-Gaceva G, Valić S, Wießner S, Heinrich G. 2017. Benefits of hybrid nano-filler networking between organically modified montmorillonite and carbon nanotubes in natural rubber: experiments and theoretical interpretations. Applied Clay Science. 136: 192-1198.

Biography (150 word limit)

Three years of working research experience at the Research Centre for Environment and Materials in Macedonian Academy of Sciences and Arts in Skopje. Specialized in elastomeric nanocomposites. Published 6 research papers in scientific journals and one book chapter.

E mail: aivanoska@manu.edu.mk

NOTES/ COMMENTS: