# Title: High pressure synthesis of new superhard nanostructured boron compounds

Yann Le Godec, Ph D, IMPMC, CNRS, UPMC Sorbonne Université, France.

Abstract (300 word limit):

Superhard materials are widely employed in industry for production of cutting and polishing tools, as well as anti-abrasive coatings. The synthesis of man-made superhard phases started from the early 1950s, as soon as the development of high-pressure techniques allowed reaching the pressures necessary for diamond synthesis. However, the experimental and theoretical design of novel superhard materials still remains a great challenge to materials scientists.

To this aim, experimental observations and simulations suggest that for many polycrystalline materials there is an optimal grain size (usually in the range of dozens of nanometers) which results in a significant, up to 50–70%, increase of hardness of the material in comparison with that of its coarse grain counterpart. This increase in hardness is known as the Hall–Petch effect. That's why the synthesis of bulk nanostructured materials remains the least-explored but challenging domain that allows combination of desired physical, chemical, and mechanical properties and gives rise to nanoelectronics, nanomechanics, bandgap engineering, etc.

Boron-based materials comprise many covalent diamond-like and boron-rich compounds whose structures are three-dimensional nets of short and strong covalent bonds responsible for their extreme hardness. In this talk, I will show that the use of very high pressures and temperatures combined with the in situ probe by X-ray diffraction with synchrotron radiation is the methodological key to control the composition and microstructure of new bulk nanostructured and superhard boron compounds and I will give many examples from our recent studies, leading to patent some new strategic compounds [1-5].

Recent Publications

- Y. Le Godec, M. Mezouar, D. Andrault, V. Solozhenko, O. O. Kurakevych, BORON CARBIDE AND METHOD FOR MAKING SAME, European patent 08787929.2 (publication 22.06.2011, Bulletin 2011/25) (2011).

- Y. Le Godec, V. Solozhenko, O. O. Kurakevych, N. Dubrovinskaia, L. Dubrovinski, NANOSCALE BORON NITRIDE, US patent USPTO 20110230122 (publication 22.09.2011, Bulletin US2011/0230122A1) (2011).

- Y. Le Godec, V.Mukhanov, V.L. Solozhenko, P. Sokolov "rapid and economical process for the production of boron phosphide", European Patent EP13306838.7 (2013).

- Y. Le Godec, V. Mukhanov, D. Vrel, P. Sokolov, V.L. Solozhenko “Mechanochemical process for the production of BP, B12P2 and mixtures thereof”, European Patent EP15305763.0 (2015).

- J.P. Itié, E. Girard, N. Guignot, Y. Le Godec and M. Mezouar. Crystallography under high pressure using synchrotron radiation, J. Phys. D: Appl. Phys. 48, 504007 (2015).

Biography (150 word limit)

Dr. Yann Le Godec, CNRS researcher, is an experienced user of large-scale instruments (ESRF, SOLEIL, DESY, DIAMOND, ILL, ISIS, etc.) for high pressure research. During last years, he has used high pressure to create new solids with advanced mechanical and physical properties. His activities led him to patent numerous new dense materials with light elements that have aroused considerable interest in search for novel superhard phases all over the world. He has been also leading in the development of in situ high pressure - high temperature diffraction techniques, both for x-rays and neutron studies. For his works, Dr. Le Godec received the CNRS "medaille de bronze" Award.

E mail: yann.legodec@impmc.upmc.fr