Title: Synthesis & Characterization of Cobalt –Graphene Nano Composite by Mechanical Milling and Sonicated Exfoliation

Krishna Kumar, Department of Metallurgical & Materials Engg., Malaviya National Institute of Technology Jaipur, India.

Upender Pandel, PhD, Department of Metallurgical & Materials Engg., Malaviya National Institute of Technology Jaipur, India.

M.K. Banerjee, PhD, Department of Metallurgical & Materials Engg., Malaviya National Institute of Technology Jaipur, India.

Email id: kkumar.meta@mnit.ac.in

Abstract:

A Cobalt nanoparticle – Graphene composite was synthesized by using mechanical milling and sonication assisted exfoliation. Graphite powder and Cobalt metal powder  in the ratio of 4:1 by weight was first mechanically milled for 60 hrs in Toluene  medium. The milled powder was then exfoliated by using sodium lauryl sulfate surfactant to produce Cobalt – Graphene nano composite. The composite have shown good magnetic property due to the presence of Cobalt nanocrystals and this was confirmed by Vibrating Sample magnetometer (VSM). Due to the sonication-assisted exfoliation, few layers of Graphene formed which were confirmed by Raman Spectroscopy, X-ray Photo Electron Spectroscope (XPS)  and Atomic Force Microscopy. Phase analysis and Size of the Cobalt – Graphene nano composite was calculated with the help  of X-Ray Diffraction pattern and Transmission Electron Microscopy.