Abstract

Abstract Title: Nanostructured sensors for colorectal cancer screening device

G. Zonta1,2, N. Landini1.2, C. Malagù1,2

1 University of Ferrara, Via Savonarola 9 - 44121 Ferrara, Italy

2 SCENT S.r.l., Via Quadrifoglio 11 – 44124 Ferrara, Italy

Abstract (not more than 300 words)

Among the major goals of medicine there is the preventive screening of tumors, in order to prevent their degeneration. Colorectal cancer (CRC) shows a curability rate up to 90%, if identified at stage I. The work presented here started in 2013 with the reproduction of an artificial intestine inside a laboratory set-up, in order to find the best sensor array capable of recognizing CRC-gaseous biomarkers produced by tumor cells inside a mixture of intestinal interferers. After that, the approach changed, moving towards the analysis of fecal exhalations. After a feasibility study, that lead to the foundation of the start-up SCENT in 2015, the most efficient sensors combination was chosen. This study was conducted in collaboration with Hospital S. Anna of Ferrara, that provided the stool samples of people affected by CRC during surgery. Controls were healthy volunteers. This passage was fundamental to proof the recognition capability of sensors inside a portable device (SCENT A1), patented in Europe, composed of a core of five nanostructured sensors, a pneumatic system and a specific electronics.  After having obtained the acceptance by the ethics committee, a clinical validation protocol started in May 2016, to demonstrate the capability of SCENT A1 of identifying the difference between fecal exhalation of healthy and CRC-affected subjects. The protocol will end in 2019 and involves the Hospital S. Anna and AUSL of Ferrara and Ospedale del Delta of Lagosanto. The tests are compared to the results of fecal occult blood test (FOBT) using colonoscopy as a gold standard. A specific algorithm of analysis has been realized for data classification. On 100 comparisons, the method correctly classified the 90% of healthy subjects, the 100% of CRC-affected and the 57% of low risk adenomas. If combined with FOBT, our test will considerably improve specificity, eliminating a huge number of non-operative colonoscopies.

Keywords—Colorectal Cancer, Gas Sensors, Nanostructure, Preventive Screening

Professional Biography (100-150 words)

Giulia Zonta was born in Italy in 1988 and has completed her PhD in Matter Physics in April 2017 at the University of Ferrara. She is now working as a postdoctoral fellow at the Department of Physics and Earth Sciences, working with the Sensors Laboratory team, coordinated by Prof Cesare Malagù. She is the Sales Executive and co-founder of the start-up SCENT S.r.l, with the aim of realizing prototypes for tumour preventive screening and monitoring. She published on reputed international journals and she is a reviewer. She is assistant in General Physic course.

Email Id: giulia.zonta@unife.it