Flexible free-standing composite films having 3D continuous structures of hollow graphene ellipsoids:

Author Name: Eun Goo Lee1,2, Jonghwi Lee1, and Sang-Soo Lee2

Affiliation: 1Dep. of Chem. Eng. and Mater. Sci., Chung-Ang University

2Photo-Elect. Hyb. Res. Center, Korea Institute of Science and Technology

Country:  Republic of Korea

Abstract:
Recently, the use of three-dimensional (3-D) graphene networks in polymer matrices has attracted a great deal of interest as a new strategy for fabricating highly flexible and conductive graphene-based polymer nanocomposites. In the present work we investigated 3-D continuous network structures based on graphene ellipsoids and the effects of their pore morphology on the properties of their polymer composites. For that matter, the hollow graphene ellipsoids and the free-standing graphene films based on 3-D continuous structures of hollow graphene ellipsoids were fabricated and their properties were examined in terms of the structural factor. In order to obtain graphene ellipsoids, positively charged polystyrene (PS) spheres were first wrapped with negatively charged graphene upon simple mixing, and their core-shell assembly of spherical nature were transformed to ellipsoids by unidirectional stretching. Graphene-wrapped PS ellipsoids were assembled into films by simple jet-spray coating, and finally, careful calcination of the graphene-PS assembly film resulted in successful formation of 3-D network of hollow graphene ellipsoids. Due to their improved continuity, composites based on the ellipsoids have lower sheet resistances than those based on spherical nanoparticles. Upon folding and application of pressure, composites based on the hollow graphene ellipsoids exhibited superior electrical conductivity and structural stability owing to their high mechanical strength and effective electron transport pathway. The ability to control the face-contact structures of graphene in a polymer matrix by means of particle morphology represents an effective strategy for future composite engineering.

Biography:

Dr. Sang-Soo Lee is a Principal Research Scientist/Head in the Photo-Electronic Hybrids Research Center, Korea Institute of Science and Technology, Republic of Korea. He received the BS degrees in Chemical Technology from Seoul National University in 1990, and the PhD degree in Chemical Technology from Seoul National University in 1995. He was a research fellow in Material Science and Engineering in University of Michigan, Ann Arbor, USA from 1995 to 1997.

Research Interests:

His research focuses on the development of novel materials and devices for electron/ion transport-based interconnection and renewable energy conversion.

References:

Mail Id: s-slee@kist.re.kr

Contact Number: 82-2-958-5356

Suggestions of colleagues/Professors/scholars to send invitation (if any)

Suggestions of Sessions in the conference (if any)