Photograph

Title of the Abstract: Size and morphology controlling of ethylcellulose nanoparticles by using different surface active polysaccharides in the emulsion evaporation preparation method

Manja Kurečič1,2,3, Božič Mojca1,2, Matej Bračič 1, Rupert Kargl1,2,3, Thomas Elschner1, Doris Tkaučič1, Silvo Hribernik1,2, Karin Stana-Kleinschek1,2,3

1Laboratory for Characterization and Processing of Polymers (LCCP), Faculty of Mechanical Engineering, University of Maribor, Smetanova 17, 2000 Maribor, Slovenia

2Institute of automation, Faculty of Electrical Engineering and Computer Science, University of Maribor, Koroška cesta 46, 2000 Maribor, Slovenia

3Institute for Chemistry and Technology of Materials (ICTM), Graz University of Technology, Stremayrgasse 9, 8010 Graz, Austria

Abstract:

Cellulose ethers, e.g., ethylcellulose (EC) was discovered in 1912 with the work of Lilienfeld (US patent 1188376 A). EC and ethylhydroxyethylcellulose are the only industrially significant organosoluble cellulose ether derivate being commercially produced for around ninety years now. Despite of their wide usages in plastics, as a pigment grinding base, as flexible coatings for paper, cloth, and leather, as an electrical insulant, etc. there is still huge research going on in regards to the EC exploitation for new commercial applications. In the recent years a special attention was paid to design new methods for producing a polysaccharide nanostructured assemblies that can possess properties that are significantly different from the bulk material. Especially cellulosic based nanoparticles exhibit a great potential as holders for imaging and drug agents because of their intrinsic physicochemical properties being biocompatible and biodegradable.

In this study, we applied an emulsification-evaporation method to manufacture the EC nanoparticles. Ethyl acetate was used as water immiscible, volatile organic solvent and as a stabilizing surfactant in water polyvinyl alcohol and different surface active polysaccharide (i.e. carboxymethyl cellulose, hydroxyethyl cellulose, high molecular weight methylcellulose and low molecular weight methylcellulose) were examined. To control particle properties, the nanoemulsion template was regulated by adapting the concentration of the surface active polysaccharides. Newly produced EC nanoparticles were characterized for particle size by particle size analyzer, stability by zetasizer and surface morphology by FE-SEM.

The authors would like to acknowledge funding from the European Union’s Horizon 2020 research and innovation program under grant agreement No 665172 (Conquer - http://conquer.at/ and financial support from the Slovenian Research Agency for Program Group: textile chemistry (P2-0118).

Biography:

Dr. Manja Kurečič is an Scientific Associate at the Faculty of Mechanical Engineering, University of Maribor and Technical University Graz, Austria, where she is working on several national and international basic and applied projects. She has a background on nanocomposite hydrogels for water purification, which she upgraded with new technologies for preparation of different nanostructured materials that can find applications in technical also well as in biomedical area. Lately she is focusing on development of innovative nanofibrous materials by using electrospinning method.

Mail: manja.kurecic@um.si