Photograph

## Title of the Abstract: Plastic Scintillator, exploration of photo-physical properties and chemical optimisation

Author Name: Dr BERTRAND Guillaume

Affiliation: LCAE (Sensor and Electronic Architecture Laboratory)

University: CEA Saclay /DRT/LIST/DM2I/LCAE

Country: France

# Abstract:

Thanks to a good ratio between cost and performances, plastic scintillators are now in the center of radiation detection applications. But due to the fact that most of the formulation were created in the 50’s, there is now a huge incentive in creating new recipes, due to increasing need of large scale detectors.

The conversion of high energy ionisation to a visible photons emission is call scintillation. This process implied charge and exciton transport inside the bulk of the plastic. Different input ionisation (eg. alpha, beta, gamma and neutron) give different ionisation trace and can lead to slight modification of the emission characteristics, hence identification of the ionising particle. We are presenting here a comprehensive model of gamma and neutron interaction in plastic scintillators and the photophysic mechanism associated with gamma/neutron discrimination, with a particular emphasis on excited triplet state diffusion and recombination inside an amorphous polystyrene matrices.

More than the model, several chemical modification of the polymer matrix as well as additives can enhance different scintillation properties. More particularly addition of metal additives, high concentration of fluorophores, modification of the polymeric matrices and addition of cross-linking agent are essential to specialized the plastic toward a particular application. Here again understanding charge and exciton transfers are central to optimize formulation. One of the main example is the choices of heavy metal additives for enhancing properties. We will present here our work on organometallic insertion inside plastic matrices and optimisation of the formulation towards better scintillation properties.

# Biography:

Dr BERTRAND Guillaume is a researcher at CEA Saclay near Paris, in France. Organic chemist by formation, he likes to applied chemical design to materials science and more particularly light/matter interaction. He has several centre of interest (photovoltaic, organometallic and inorganic chemistry, porous materials …) and he is now specialized in design and optimisation of plastic scintillator for nuclear instrumentation.

Research Interests: Polymer chemistry, Plastic scintillator, Photophysic, Organometallics

References:

Mail Id: guillaume.bertrand@cea.fr

Contact Number: +33 1 69 08 91 48

Suggestions of Sessions in the conference: Nanotechnology (track 12) might be the most suited, but track 7 and 3 could also be appropriate.