A study of the flow behaviour of natural rubber latex/singlewalled carbon nanotubes blends using rotational viscometry and power law model

Author: Victor Chike Agbakoba,

Supervised by: Percy Hlangothi (PhD)

Department of Chemistry, Faculty of Science.

Centre for Rubber Science & Technology, Nelson Mandela University, University way, PO Box 77000, Port Elizabeth, South Africa.

ABSTRACT:

The physiochemical modification of natural rubber latex (NRL) is achieved via the addition of finely dispersed reinforcing fillers (RFs) and chemical ingredients[1]. In recent times, singlewalled carbon nanotubes (SWCNT) have shown great prospect as a suitable RF for NRL even at very low loadings; compared to conventional RFs such as carbon black and fine particle clays.[2]

Although SWCNT are generally insoluble in water, optimised dispersions are often prepared via various physiochemical treatments which promote the exfoliation of the otherwise entangled SWCNT[3]. Chemical modification (covalent methods) involves the introduction of functional groups on the surfaces of SWCNT. Whereas, physical treatment (non-covalent methods) often utilise surfactant systems with a relatively high Hydrophile-Lyphophyle balance (HLB).[4]

It thus is pertinent to elucidate how the incorporation of dispersed SWCNT affects the flow behaviour of NRL. This is because, the flow behaviour of NRL plays a huge role during storage, handling and processing [5]. In this work, the flow behaviour of NRL/SWCNT blends prepared via the latex stage mix method have been investigated.

Flow behaviour was studied according to the principles of rotational viscometry on a Modular Compact Rheometer (MCR) fitted with a concentric cylinder geometric measuring system. The experimental conditions involved the exposure of blends to varying shear rates (0.1 – 100 s-1) at three isothermal temperatures (25, 30 and 35 °C); this was based on the probable conditions during storage, handling and processing of rubber lattices.

Results obtained revealed high apparent viscosity at low shear rates for samples with higher loadings of SWCNTs. For Instance, viscosity at 25 °C and 1 s-1 of blends with 0.08% SWCNT was 2.5 Pa.s whilst that with 0.02% loading 0.49 Pa.s. Again, characteristic shear thinning behaviour was observed, which was confirmed by the Power Law model fits.

REFERENCES

[1]	D. C. Blackley, Polymer Latices: Science and Technology Volume 3: Applications of Latices: Springer Science & Business Media, 2012.

[2]	G. Sui, W. Zhong, X. Yang, and Y. Yu, "Curing kinetics and mechanical behavior of natural rubber reinforced with pretreated carbon nanotubes," Materials Science and Engineering: A, vol. 485, pp. 524-531, 2008.

[3]	D. Ponnamma, S. H. Sung, J. S. Hong, K. H. Ahn, K. Varughese, and S. Thomas, "Influence of non-covalent functionalization of carbon nanotubes on the rheological behavior of natural rubber latex nanocomposites," European Polymer Journal, vol. 53, pp. 147-159, 2014.

[4]	L. Vaisman, H. D. Wagner, and G. Marom, "The role of surfactants in dispersion of carbon nanotubes," Advances in colloid and interface science, vol. 128, pp. 37-46, 2006.

[5]	H. L. Corrêa, A. M. F. d. Sousa, and C. R. G. Furtado, "Natural rubber latex: determination and interpretation of flow curves," Polímeros, vol. 25, pp. 365-370, 2015.

AUTHOR BIOGRAPHY

Victor Chike Agbakoba was born in the north eastern state of Adamawa, north-east Nigeria. He first completed a Bachelor of Science Honours (BSc. Hons) degree in Chemistry from the University of Abuja, Nigeria in the year 2013. He then proceeded to acquire another BSc. Hons and subsequently a Master’s of science degree in chemistry at Nelson Mandela University (South Africa). Currently, Victor is a first year PhD. Chemistry candidate at the same institution. Some of his hobbies includes, photography, swimming and a nice game of squash.

s215161440@mandela.ac.za / vic.dave@yahoo.com