SUGARCANE BAGASSE AND PEANUT SHELL FOR THE SYNTHESIS OF MAGNETIC BIO-COMPOSITES AS ADSORBENT MATERIALS FOR PESTICIDES REMOVAL

Helen Paola/Toledo Jaldin1,2, Víctor Sánchez Mendieta2, Alien/Blanco Flores3,4, Gustavo/López Téllez2, Alfredo Rafael/Vilchis Nestor2

1 Posgrado en Ciencia de Materiales (Universidad Autónoma del Estado de México, México)

2 Centro Conjunto de Investigación en Química Sustentable UAEM-UNAM, (CCIQS, México)

3 Ingeniería Mecánica (Tecnológico de estudios superiores de Tianguistenco, México)

4 Instituto de Metalurgia (Universidad Autónoma de San Luis Potosí, México)

Abstract

In the present study two industrial wastes were recycled in order to synthetized two magnetic bio-composites for the removal of the pesticides carbofuran and iprodione from aqueous solutions. Sugarcane bagasse and peanut shell are common agro industrial wastes in Mexico. They barely have applications as raw material for industrial processes. Despite of being biodegradable materials their generation in large quantities make them environmental liabilities that create pollution. Composites magnetite/sugarcane-bagasse (MBO) and magnetite/peanut-shell (MPSo) were synthetized and applied as adsorbent materials. MBo and MPSo were synthesized by a co-precipitation process. They were characterized by Raman spectroscopy, transmission electron microscopy, scanning electron microscope and Fourier transform infrared spectroscopy. The removal was carried out by the batch adsorption process under the same conditions of concentration, amount of adsorbent, temperature and contact time. The spectrophotometric technique was adopted for the estimation of concentration of pesticides before and after the adsorption. The kinetic data were applied on pseudo-first order, pseudo-second order and Elovich models. The Elovich model described better all systems.  The isotherm models applied were Langmuir, Freundlich, Sip, Temkin, Redlich-Peterson and Dubinin–Radushkevich. Isotherms experimental data of carbofuran and iprodione can be best described by more than one model, according to the statistics parameters, but in general, the process took place by a chemical adsorption since Sip model was a common equation in all systems. The maximum adsorption capacities for carbofuran were 175 and 89.3 mg/g, for iprodione were 119 and 2.76 mg/g for MBo and MPSo, respectively.

Keywords— Agro industrial waste, carbofuran and iprodione adsorption, magnetic bio-composite, sugarcane bagasse and peanut shell.

Professional Biography (100-150 words)

Helen Paola Toledo Jaldin has completed his Master degree at Technological University of Toluca, México and made a master course of Development of Environmental Projects at the International Institute for Environmental Education, Spain.  She spend three months in Manchester University after she won a mobility scholarship. Where she has won the Second Prize for Multidisciplinary Research, Product Development and Business Proposal Project in United Kingdom, 2015. At the moment she is making the PhD at the Universidad Autónoma del Estado de México. She obtained a scholarship from the Mexico government for Master and PhD students. She has published 5 papers from 2015 in reputed journals.

Email Id: helenpao@hotmail.com

Any Comments: