Nutrient Removal Efficiency of A2O Wastewater Treatment System at Varying Conditions of Hydraulic Retention Time

Tahir Imran Qureshi

Department of Environmental Engineering, NFC-Institute of Engineering & Technology Multan, Pakistan

Abstract

This study aimed at analyzing the nutrient removal efficiency corresponding to the variations in hydraulic retention time (HRT) using Anaerobic-Anoxic-Aerobic (A2O) process as advanced wastewater treatment system. The process was optimized for maximum removal of organic matter in the influent in meeting growing stringent quality standards of wastewater discharge. BOD5/TKN ratio was recorded at 2.8 that suggested at lower value than the general single step nitrification process. A minimum target of 90% BOD5 removal efficiency was achieved at all conditions of HRT. No big difference in BOD5 removal was recorded at varying HRT, however, the rate was increased as F/M ratio was decreased depending on an extended HRT conditions.  As the T-N removal was concerned, it was reduced with decreasing HRT value. Nitrification rate was found increased along with increasing NH4-N loading rate; however, nitrification rate was decreased by reducing HRT at identical loading rate. A high value of BOD5/P ratio recorded in the experiment suggested an enhanced denitrogenation and phosphorous uptake ability by micro algae. Therefore, it is concluded that HRT and concentration of total dissolved solid (TDS) including ammonia nitrogen and total phosphorous have no influence on the removal rate of organic fraction but partly affects the adaptation period of microorganism in the system.

Keywords: Biological nutrient removal, F/M ratio, nitrification and denitrificaion

Professional Biography:

Dr. Tahir Imran Qureshi has completed his Ph.D. in Environmental Engineering from Kyungpook National University, South Korea in 2003. Currently, he is working as Professor/Director, Department of Chemical Engineering at NFC-Institute of Engineering & Technology, Multan, Pakistan. His field of specialization is solid waste management, wastewater recycling, leachate treatment, resource recycling, conservation, Environmental Management Systems, and drinking water quality issues. He has published more than 30 scientific papers in well reputed journals in the field of environmental engineering and sciences. He is a sole author of a book entitled, “Drinking Water: Its Impurities and Treatment” published by Higher Education Commission of Pakistan.

Presenting author details

Full Name: Tahir Imran Qureshi

Contact number: 92-300-6334429

Email: tahirimran@nfciet.edu.pk