Abstract

Abstract Title: Holistic zero effluents - low energy consuming recycling technology turning agricultural farm waste to viable granulated organic fertilizer

Bar Eytan, Li Xu, Yang Fenglin

Guangzhou EWA Technologies, Guangzhou, China

Abstract (not more than 300 words)

Agricultural farms produce various groups of solid waste, including plant material, animal manure, animal corpses, plastic waste etc. Recycling of such waste make use of

holistic waste recycling management means considering each type of waste as resource for energy or new fine product and thereby minimize energy consumption, eliminate effluents (liquid, solid and gas) and maximize the value of the recycled products.

The energy source for the system came from low temperature pyrolysis system, turning plastic and organic waste to syngas and diesel in singlee stage, the syngas consumes to heat the pyrolysis reactor and the diesel produce use to operate electric generator while waste heat from the pyrolysis is use to dry the final product, for animal corpses disinfection and evaporate leachate.

In parallel to the pyrolysis system, manure, straw, animal corpses and waste of rendering plant are all recycled to high value N rich granulate organic fertilizer product, make use of low thermal consuming and leachate free process combining natural Ca based components together with coal powder. The process controlled by AI system, that includes 24h composting-disinfection stage, robotic turning, spreading and discharge system at 15 day process, automatic mixing-adjusting system to ensure stable product quality and granulation without heating. The composting and granulating systems make use waste heat and electricity originated by the pyrolysis part of the system.

The holistic organic waste recycling management is currently operating at scales of 20 and 150T/day organic waste, produce 8.5 to 72 ton granulated organic fertilizer per day with zero effluents in Europe, China and the Philippines.

Keywords—Pyrolysis, Manure, Compost, Organic fertilizer

Professional Biography (100-150 words)

Dr. Eytan Bar, CEO of Guangzhou EWA Technologies, graduated by the Hebrew University of Jerusalem, Israel, followed by Post-Doc at Max Plank institute of Plant Biology, Koln, Germany. Dr. Bar established the studies of Environmental Engineering in Ben-Gurion University of the Negev, Israel and serves as the Head for the Department until 2010. Dr. Bar invented 2 Cutting-edge technologies for extraction of water from air in arid climate and medical waste treatment and recycling. Since the year 2016 Dr. Bar serves as Expert Talent for advanced Environmental technologies of Guangzhou City and Guangdong Province, China. During his activities in China Dr. Bar received 7 awards for his achievement in Cleantech, mainly for on-site medical waste treatment, pyrolysis for waste to energy and fast composting.

Paste your Photograph here

Email Id: bar@gewatech.com

Any Comments: