# Production of methyl levulinate and levulinic acid by direct conversion of α-cellulose and raw biomass using a cheap homogeneous catalyst

Maricelly Martinez, PhD student, University of Sherbrooke, Canada

Xavier Duret, PhD, University of Sherbrooke, Canada Thierry Ghislain, PhD, University of Sherbrooke, Canada

Doan Pham, PhD, University of Toulouse, France

Ange Nzihou, Pr University of Toulouse, France

Jean-Michel Lavoie, Pr University of Sherbrooke, Canada

### Abstract

Production of methyl levulinate (ML) and levulinic acid (LA) through a direct conversion of α-cellulose and raw biomass in methanol was realized. The effect of acid concentration (0.04-0.24 mol/L), reaction time (0.5-7 hours), cellulose concentration (2.5-12% wt.) and temperature (180-200°C) for the conversion of α-cellulose and raw biomass to methyl levulinate and levulinic acid was analyzed with a central composite design (CCD) using the response surface methodology (RSM). Three dependent responses (ML yield, LA yield and residues yield) were studied to determine the optimum combination of the four factors. Results showed that the optimal conditions involved an acid concentration of 0.1796 mol/L, a biomass concentration of 2.5 wt% and a reaction time of 4.21 hours at 200°C. These conditions lead to a theoretical yield of methyl levulinate and levulinic acid of 68.14% and 14.30% respectively and experimental yield for methyl levulinate, levulinic acid and residues yield were 62.0%, 15.95% and 11.2% respectively.

### Recent Publications

- Martínez M., Cortés F., Franco C. (2013) Tratamiento de agua basado en la adsorción de crudo en nanopartículas polares y no polares. Informador técnico, SENA, Vol. 77 (1). ISSN 0122-056X-eISSN 2256-5035. Pag. 59-68.

- Cortés F., Ruiz M., Franco C., Martínez M. (2013) Tratamiento de agua basado en la adsorción de crudo en nanopartículas polares y no polares. XV Congreso Colombiano del Petróleo. ACIPET. Bogotá D.C. Colombia.

- Cortés F., Ruiz M., Franco C., Martínez M., Benjumea P., Patiño E. (2014) Water Remediation Based on Oil Adsorption Using Nanosilicates Functionalized with a Petroleum Vacuum Residue. Adsorption Science and Technology Vol. 32 No. 2-3.

- Martínez M., Duret X., Ghislain T., Pham Minh D., Nzihou A., Lavoie J.M. Methyl levulinate and levulinic acid production by direct conversion of softwood bark using a metal salt as catalyst. (In review).

- Martínez M., Pham Minh D., Nzihou A., Lavoie J.M. Conversion of methyl levulinate to levulinic acid using copper-based catalysts. (In review).

Biography

Maricelly Martínez Aguilar is native from Bogota, Colombia and she holds a bachelor’s degree in Chemical Engineering from the University of America at Bogotá since 2012 and a Master's degree in Petroleum Engineering at the National University of Colombia at Medellin in 2015. Maricelly has the experience of the production of bioproducts for the energy field. She Joined the Biomass technology laboratory of University of Sherbrooke in Canadá in August 2016 to do a Ph.D on the conversion of sugars of biomass into biobutanol by a thermochemical process.

Email: maricelly.martinez.aguilar@USherbrooke.ca

Presenting author details

Full name: Maricelly Martínez Aguilar

Contact number: +15145687117

Mail id: maricelly.martinez.aguilar@USherbrooke.ca

Category: oral presentation