Photograph

Title of the Abstract: Acupuncture Sample Injection for Microchip Capillary Electrophoresis and Electrokinetic Chromatography

Presenting Author Name: Ji Won Ha

Affiliation: Chemistry

University: University of Ulsan

Country: South Korea

Scientific Session name / number: Track 1 recent developments in separation techniques

Category: (Oral presentation / Poster presentation): Poster

Abstract:
A simple nanoliter-scale injection technique was developed for polydimethylsiloxane (PDMS) microfluidic devices to form the well-defined sample plugs in microfluidic channels. Sample injection was achieved by performing acupuncture on a channel with a needle and applying external pressure to a syringe. This technique allowed us to achieve reproducible injection of a 3-nL segment into a microchannel for PDMS microchip-based capillary electrophoresis (CE). Capillary zone electrophoresis (CZE) and capillary electrochromatography (CEC) with bead packing were successfully performed by applying a single potential in the most simplified straight channel. The advantages of this acupuncture injection over the electrokinetic injection in microchip CE include capability of minimizing sample loss and voltage control hardware, capability of serial injections of different sample solutions into a same microchannel, capability of injecting sample plugs into any position of a microchannel, independence on sample solutions during the loading step, and ease in making microchips due to the straight channel, etc.

Biography:

Prof. Ji Won Ha is a associate professor at the Department of Chemistry, University of Ulsan since 2015. Dr. Ha majored in Analytical Chemistry and obtained a Ph.D. from Iowa State University in 2013.

Research Interests: Single molecule nanoparticle spectroscopy, Nanobiophotonics, Microchip separation

Mail Id:

Contact Number: 82-10-4634-6141

Twitter account:

Linked In account:

Facebook account:

Suggestions of colleagues/Professors/scholars to send invitation (if any)

Suggestions of Sessions in the conference (if any)