Arsenic speciation by using new syntheses polymeric material and solid phase microextraction and its graphite furnace atomic absorption spectrometric determination in water samples

Jamshed Alia,b, Mustafa Tuzena, Tasneem Gul Kazib, Baki Hazerc

### aGaziosmanpasa University, Faculty of Science and Arts, Chemistry Department, 60250 Tokat, Turkey

bNational Centre of Excellence in Analytical Chemistry, University of Sindh, Jamshoro 76080, Pakistan

cBulent Ecevit University, Chemistry Department, 67100 Zonguldak, Turkey

Total concentration does not give enough information about toxicity and bioavailability of arsenic in water and environmental samples. Arsenic present in different oxidation states in aqueous solution but it mostly exist in As(III) and As(V). The toxicity of As(III) is higher than As(V). So, As(III) and As(V) speciation in water samples is very important for environmental studies (1,2).

New syntheses polystyrene polydimethyl siloxane polymer was loaded into the micropipette tip of the syringe system for solid phase microextraction of arsenic(III) and arsenic(V) speciation.

Various analytical parameters such as pH, eluent type and its volume, polymer amount, sample volume, etc. were optimized. Effect of some cations, anions and transition metal ions were also investigated. While the recovery of As(V) was found quantitative in the pH range of 6-8, As(III) recovery was found below 5% in all pH values. The total concentration of arsenic was obtained by the addition of oxidizing agent potassium permanganate. The accuracy of the developed method was confirmed by using certified reference materials. The relative standard deviation was found lower than 5 %. Low detection limit and high preconcentration factor were obtained according to literature values. Optimized method was successfully applied to natural water samples for the speciation of As(III) and As(V). Graphite furnace atomic absorption spectrometry was used for the determination of arsenic concentration.

### References

- Ali, J., Tuzen, M., Kazi, T.G., Hazer, B., Talanta, 2016, 161, 450-458.

- Wu, H., Wang, X., Liu, B., Liu, Y., Li, S., Lu, J., Tian, J., Zhao, W., Yang, Z., Spectrochimica Acta Part B: Atomic Spectroscopy, 2011, 66, 74-80.

Acknowledgements

Jamshed Ali is grateful to the TUBITAK (Scientific and Technological Research Council of Turkey) for being awarded Research Fellowship Program of 2216 and providing financial support. Dr. Mustafa Tuzen thanks to Turkish Academy of Sciences for financial support.

### E-mail: mustafa.tuzen@gop.edu.tr