Paste your logo here

Comparison of two Ba-faujasite adsorbents for separation of xylene isomers

Maryam Ahmadipour a, Mohammadreza Khosravi-Nikou a*, Ahmad Shariati a, Shahram Saadi b

a Department of Gas Engineering, Ahvaz Faculty of Petroleum, Petroleum University of Technology, Ahvaz, Iran

b BouAli Sina Petrochemical Co., Mahshahr, Iran

Presenting Author Name:

Affiliation: Mohammadreza Khosravi-Nikou

University: Petroleum University of Technology

Country: Iran

Scientific Session name / number: Adsorption

Category: (Oral presentation / Poster presentation/ either)

Abstract

In this work, the equilibrium adsorption of xylenes, including para xylene, meta xylene, ortho xylene and ethyl benzene in liquid phase of mesitylene solvent onto two types of commercial adsorbents (SPX3000-fine and SPX3000-coarse) was studied. The specifications of adsorbents were specified by using various characterization techniques like X-ray Fluorescence (XRF), scanning electron microscopy (SEM), thermogravimetric analysis (TGA). The adsorption capacity and removal percentage of adsorbents were determined at ambient temperature and under atmospheric pressure. It was discovered that SPX3000-fine particles brought out higher adsorption capacity than SPX3000-coarse for removing para xylene component. The outcomes of present experiments show that SPX3000-fine particles have higher degree of removal percentage of para xylene than other adsorbent. Several isotherm models were applied to describe the adsorption processes, where Langmuir model reveals satisfactory output for covering experimental data of entire components.

Biography:

Mohammad Reza Khosravi-Nikou received his PhD in Chemical Engineering from Isfahan University of Technology, Iran. From 2008 till the present he works as a faculty member (associate professor) of Gas Engineering Department in Petroleum University of Technology, Iran. He published several researches in the field of heavy metals removal from waste water and liquid hydrocarbon fuel desulfurization and denitrogenation.

Research Interests: Adsorption

Keywords: Adsorption, Para-xylene separation, Xylene isomers

Mail Id: mr.khosravi@put.ac.ir

Contact Number: 00989163103740

Twitter account: (If interested)

Linked In account: (If interested)

Facebook account: (If interested)

Suggestions of colleagues/Professors/scholars to send invitation (if any)

Suggestions of Sessions in the conference (if any)

registration fee of the registered author is received.