Bioinspired wettability surfaces: Development in micro- and nanostructures

Yongmei Zheng

Beihang University (BUAA), Beijing, 100191 (P. R. China)

Abstract (not more than 300 words)

Bioinspired novel surfaces are developed by understanding wettability mechanism of natural biological functional surfaces (e.g., spider silk, cactus spine, beetle back, butterfly wing, lotus leaf, etc.). In our investigation, the cooperate between of various-style gradient micro- and nanostructures (MN) greatly provide into excellent functions of biological and bioinspired surfaces. By understanding a combination of multiple gradients in a periodic spindle-knot structure take on surface of wetted spider silk for highly efficient water collection. The series of functional bioinspired wettability silks have been designed and fabricated by using integrative methods such as dip-coating, fluid-coating, to combine the Rayleigh instability theory. The as-fabricated silk can be opened in the functions such as fog harvesting, droplet transport, etc. Besides, some bioinspired surfaces can be developed by understanding the gradient effects on butterfly wing and lotus leaves. Thus bioinspired wettability surfaces with ratchet MN, flexible lotus-like MN are fabricated successfully to achieve the functions such as anti-icing, ice-phobic and de-ice abilities, etc. Otherwise, droplet can be manipulated on some dynamic responsive gradient surfaces. These surfaces can be of practical applications in microfluidics, engineering fog harvesting, micro-devices in aerospace, industry and so on.

Keywords—bioinspired surfaces, wettability, gradient micro- and nanostructures

Professional Biography (100-150 words)

Yongmei Zheng, PhD, is a professor at School of Chemistry, Beihang University. Research interests are focused on bioinspired surfaces with gradient micro- and nanostructures to control dynamic wettability, and develop the surfaces with characteristics of water repellency, anti-icing, or fog-harvesting, tiny droplet transport, and so on. Publications are more than 100 SCI papers included in Nature, Adv. Mater., etc., with 12 Cover stories, and a book “bioinspired wettability surfaces: Development in micro- and nanostructures” by Pan Standard Publishing, USA. Her work was highlight as scientist on News of Royal Society of Chemistry, ChemistryWorld in 2014. She is a member of Chinese Composite Materials Society (CSCM), American Chemistry Society (ACS), International Society of Bionic Engineering (ISBE), and International Association of Advanced Materials (IAAM). She wins an ISBE outstanding contribution award in 2016 by ISBE and an IAAM Medal in 2016 by IAAM, in Sweden. Her homepage: http://www.zhengyongmei.polymer.cn.

Email Id: zhengym@buaa.edu.cn

Any Comments: