Electrochemical Conversion of Energy and Chemicals Using Conductive Nanomaterials and Wettability

Ying Zhu,

Beihang Uinversity, Beijing, China

Abstract

The electrocatalytic conversion of renewable resources, such as electrical energy, CO2 to chemicals and fuels, is a promising means to meet the most urgent technological goals of our time; namely, clean energy production and environmental remediation. Some of the current challenges for modern electrocatalysis are to improve microbial electrocatalysis, oxygen reduction reaction (ORR) and CO2 electroreduction. Here, micro/nanostructured conductive materials have been developed for improvemnt of electrocatalytic activities by tuning wetting and electronic properties. PANI nanofiber arrays could act as a solid-state polymeric mediator for bacterial extracellular electron transfer (EET) by tuning PANI oxidation states. The surface wettability of the electrodes strongly influences the bacterial EET activity at a fixed external potential: the EET activity on a hydrophilic electrode is more than five times higher than that on a hydrophobic one. The hydrophobic exfoliated MoS2 (H-E-MoS2) nanosheets are fabricated from bulk MoS2 by a cost-effective ball-milling method, followed by decoration with fluorosilane. H-E-MoS2 as cost-effective electrocatalyst is capable to directly reduce CO2 and H2O to tunable syngas production with a wide range of CO/H2 ratio from 1:2 to 4:1

Keywords—Electrochemical catalysts, energy conversion, CO2 conversion, conductive nanomaterials.

Professional Biography (100-150 words)

Ying Zhu Doctoral Supervisor, Vice dean of School of Chemistry, Beihang University. She has undertaken more than 10 significant programs, including the National 863 Program, National 973 Program and National Science Foundation, and so on. She has already authorized 14 national invention patents, and 6 national and international invention patents are in publicity. She has been awarded as National Natural Science Award of China, Second Prize (2014); Beijing Science and Technology Award, First Prize (2012), New Century Excellent Talents in University, Ministry of Education (2008), K. C. Wong Post-doctoral Research Award Funding, Chinese Academy Sciences (2006).

Email Id: zhuying@buaa.edu.cn

Any Comments: