Kinetics and phase transformation of Cu-Al-Mn-Ag alloys

T. Holjevac Grgurić1*, D. Manasijević2, I. Anžel3, I. Ivanić1, S. Koužuh1, M. Gojić1

1Faculty of Metallurgy, University of Zagreb, Aleja narodnih heroja 3, Sisak, Croatia

2Technical Faculty Bor, University of Belgrade, V.J. 12, Bor, Serbia

3Faculty of Mechanical Engineering, University of Maribor, Smetanova ulica 17, Maribor, Slovenia

Abstract (not more than 300 words)

Practical exploitation of Cu-based shape memory alloys has been very intense lately due to good shape memory effect, high thermal conductivity, damping properties, superelasticity and also very important, low production costs. Cu-Al-Mn shape memory alloy shows higher ductility related to other Cu-based SMA materials, which makes them suitable for cold-workability. Cu-Al-Mn and Cu-Al-Mn-Ag alloys with different content of manganese and silver were prepared by continuous casting and by electric-arc furnace. Cu-Al-Mn alloy was further cold rolled to a thickness of 1 mm. Specimens were then betatized at 900 °C for 30 minutes and quenched in water. Microstructure of as-cast and homogenized state was determined by scanning electron microscopy (SEM-EDS) and optical microscopy (OM). The existence of crystal phases was detected by x-ray diffraction analysis (XRD). Kinetic parameters of martensite transformation of Cu-Al-Mn and Cu-Al-Mn-Ag alloys were carried out by simultaneous thermal analysis (STA DSC/TG) and damping parameters by dynamic-mechanical analysis (DMA). Results showed intensive formation of martensite structure in continuously casted alloy. Transformation temperatures and hysteresis were correlated with content of alloying element, parameters of producing technology and cold rolling.

Keywords—Cu-Al-Mn alloy, damping, martensite transformation, shape memory alloys

Acknowledgements     This work has been partially supported by Croatian Science Foundation under the project IP-2014-09-3405.

Professional Biography (100-150 words)

Associate Professor Tamara Holjevac Grgurić, PhD, earned doctoral degree in 2006. In 2009, she joined the University of Zagreb Faculty of Metallurgy as Assistant Professor and 2016. she became Associate professor.  During 2005, she participated in a scientific training at the University of North Texas in the Laboratory of Advanced and Optimized Materials, Denton, USA, and in 2016 at the Ruhr-Universitat, Interdisciplinary Centre for Advanced Materials Simulation in Bochum, Germany. In 2010 and 2018, she was Visiting Professor at the University of Belgrade, Technical Faculty in Bor. She was a Project Leader of 2 international projects, and associate at 6 national and international projects. Furthermore, she is a MC Member of EU project COST CA15107 and she was a MC Member of 2 finished EU projects, COST MP0902 and COST MP1206. She is a member of the Alloy Phase Diagram International Commision.

Paste your Photograph here

Email Id: tholjev@simet.hr

Any Comments: