Abstract

Abstract Title: Wind tunnel test of dielectric elastomer actuator for Mars airplane

Koji Fujita1, Mikio Waki2, Mitsuru Uejima3, Makoto Takeshita3, Seiki Chiba4

1 Institute of Fluid Science, Tohoku University, Japan

2Wits Inc., Japan

3Zeon Corporation, Japan

4Chiba Science Institute, Japan

Abstract

Dielectric elastomer (DE) is a relatively new transducer technology uses rubberlike polymers (elastomers) as actuator materials. The basic element of DE is a very simple structure comprised of a thin elastomer sandwiched by soft electrodes. When a voltage difference is applied between the electrodes, they are attracted to each other by Coulomb forces leading to a thickness-wise contraction and plane-wise expansion of the elastomer. At the material level, DE actuator has fast speed of response, with a high strain rate, high pressure, and power density of 1 W/g. Recently, airplanes are payed attention as a new platform for Mars exploration. The Mars airplane must be lightweight to fly using aerodynamic forces in the rarefied Martian atmosphere. Therefore light-weight and high-power actuators are required for the Mars airplane. The advantages of the DE actuators are beneficial for the Mars airplane. The DE actuators have a possibility to be used as actuators for control surfaces (i.e. ailerons, rudder, and elevator) and a propeller of the Mars airplane. This research reports a result of a wind tunnel test of a control surface actuation using DE actuator to investigate a feasibility of the DE actuators for the Mars airplane. A chord length of the wing is 160 mm, including the control surface of 55 mm. A ϕ80 mm, diaphragm-shaped DE of 0.1 g is used. Bias voltages are from 3.2 to 3.7 kV. Angles of attack are from -10 to 10 degrees. Flow velocities are 0, 10, and 15 m/s. Deflection angles of the control surface are measured. The result shows that the control surface is sufficiently actuated by DE actuator under various flow conditions.

Keywords— Actuator, Dielectric elastomer, Mars airplane, Wind tunnel, Wing

Professional Biography

Koji Fujita received his B.Sc. degree from the Tohoku University in 2010 in aerospace engineering, and he got a Ph.D. at the Tohoku University under the supervision of Professor Hiroki Nagai. His research has focused on an airplane for Mars exploration. His thesis work involves the conceptual design of the Mars airplane and aerial deployment technique. He continued this research at Japan Aerospace Exploration Agency as a Research Fellowship for Young Scientists of Japan Society for the Promotion of Science. Now he is an assistant professor of the Institute of Fluid Science at Tohoku University. He started a joint research with co-authors to utilize dielectric elastomer actuators for his airplane for Mars exploration.

Email Id: fujita.koji@tohoku.ac.jp

Any Comments: