Abstract Template

Abstract Title: AN INVESTIGATION INTO FLEXURAL BEHAVIOR OF CONCRETE FILLED STEEL TUBE (CFST)

Pramod Kumar Gupta

1 Professor, Structural Engineering, Department of Civil Engineering, Indian Institute of Technology Roorkee, Roorkee INDIA

Abstract (not more than 300 words)

Numerous studied have been carried out on Concrete Filled Steel Tubes (CFST) as columns in the past due to their advantages in structural system as well as in the economical aspect. Since CFST possess high strength and ductility they are mostly used in earthquake prone areas. The main advantage is the confining effect due to the steel tube present on the outer periphery as well as the local interaction between concrete and steel tubes which improves the local buckling of steel tube. In this paper a study has been carried on the flexural behavior of CFST. A three dimensional Finite Element model has been developed in software ABAQUS. Furthermore, the model has been validated with available experimental data in literature. Thereafter a parametric study has been done to study the effects of changing material properties, sectional parameters, length of specimens. Depth (D) of the beam specimen varied from 120 mm to 200 mm keeping the constant breadth (B) of 120 mm. In five specimens thickness of tube has been kept constant and equal to 1.6 mm and for rest of the specimen’s bottom thickness of tube increased from 1.6 mm to 3.2 mm at an interval of 0.2 mm to design the beam. For some of the specimens grades of steel and concrete were kept constant but for some of the specimen grades of steel and concrete were also varied to ascertain the effect of different strength. Parametric study was carried out for various specimens and moment vs. displacement curves were plotted for all the specimens. It was observed that by changing the grade of steel capacity was drastically improved but with the increase in bottom thickness of tube changes were very little. It was found that effect of grade of concrete is most significant on moment carrying capacity.

Keywords— ABAQUS, Concrete Filled Steel Tubes, Composite construction, Finite Element Method.

Professional Biography (100-150 words)

Dr Pramod Kumar Gupta is currently working as Professor of Structural Engineering in the Department of Civil Engineering of Indian Institute of Technology Roorkee, India. He obtained his doctoral degree in 2001 from Indian Institute of Technology Delhi, India in the area of Structural Mechanics. His areas of research are: Structural mechanics, finite element analysis, parallel computing, steel-concrete composite and large deformations. He has published more than 150 research papers in International Journals, Conferences; National Journals, Conferences. Out of 150, 80 papers are published in Journals. He has successfully handled and completed sponsored research projects of National and International status. Presently, he is working as Editorial Board member of 8 Journals. He has given more than 150 consultancy services. He has served in different societies of National importance at the level of treasurer and Secretary. He has experience of organizing short term courses, seminars, workshops and conferences as organising Secretary. Presently he is working as one of the Directors of Australian Association for Steel-Concrete Composite Structures. Recently, he was awarded with the Endeavour awards of Australian Government.

Paste your Photograph here

Full Name : . Pramod Kumar Gupta

Email Id: spramod_3@yahoo.com

Contact No: +911332-285425, +919411500841

Social Network Details: spramod_3 (FaceBook/LinkedIn):

Any Comments: