3D Printing of Complementary Pre-Polymers for Oral Delivery

T. Croitoru-Sadger1, S. Yogev1, A. Shabtay-Orbach1, B. Mizrahi1

1Technion, Faculty of Biotechnology and Food Engineering, Haifa, Israel

Abstract

Current 3D printing techniques of tablets involve destructive UV irradiation or high temperature that limit the use of sensitive drugs. Here, we describe a novel 3D printing technique based on two complementary liquid copolymers that are injected in a coordinate fashion to form a cross-linked 3D tablets. The successful synthesis of the pre-polymers were verified by 1H-NMR and gel permeation chromatography (GPC). Tablets swelled about 400% over 3 hours followed by a moderate disintegration.  In-vitro cytotoxicity studied on 3T3 fibroblast cells demonstrated the safety of the tablets for oral administration. Finally, tablets were loaded with Prednisone resulting in 100% drug loading efficiency and a controlled release. Release rate could be controlled by coating with Eudragit®. Thus, this unique method can be empowered to form an efficient oral drug delivery system with controlled release pattern and may provide desirable traits for drugs susceptible to heat or irradiation and possibly to other biomedical applications.

Keywords— 3D Printing, PEG, Self Cross-Linking, Tablets

Professional Biography

Tsuf received her B.Sc. in Chemical Engineering and her M.Sc in Biotechnology and Food Engineering, both from the Technion – Israel Institute of Technology.

She joined the laboratory for biomaterials under the supervision of Prof. Boaz Mizrahi on March 2014. Currently, she is a Ph.D. student. Her research interest focused on 3D printer, using unique printing technics, which includes injections of liquid bio-polymers.

Email Id: tsuf.cr@gmail.com